v.1.01.

TMKLL4 v4.54 (DOS) driver library,

tmk1553b.sys v2.11 (Windows NT 4.0) driver,

1553bwdm.sys v2.11 (Windows 98/ME/2000/XP) driver,

tmk1553b v2.11 (Linux kernel 2.4) driver

Reference Manual

The usability of the referenced driver components is defined by specifying the OS used in brackets in the component description header:

[DOS] – usability in DOS,

[Win] – usability in Windows NT 4.0 and in Windows 98/ME/2000/XP,

[Linux] – usability in Linux.

For the universal driver components (similar for all drivers) [DOS, Win, Linux] usability is not specified.

All the universal functions are defined in the DOS driver format, the ‘unsigned’ means 16 bit unsigned, the ‘unsigned long’ means 32 bit unsigned, the ‘int’ in DOS means 16 bit signed, in Windows and Linux - 32 bit signed.

**************************************************************************

Error Code Variable

**************************************************************************

tmkError

int tmkError;

If an erratic situation occurs, the tmkError will contain the code of the error detected by the driver:

TMK_BAD_TYPE – invalid TMK device type;

TMK_BAD_IRQ – invalid number of the interrupt request line;

TMK_BAD_NUMBER – invalid number of the TMK device;

TMK_BAD_FUNC – this device type does not support the called function;

TMK_PCI_ERROR – invalid attempt to configure the PCI device;

BC_BAD_BUS – the redundant bus is set for the BC without redundancy;

BC_BAD_BASE – invalid DPRAM base of the BC;

BC_BAD_LENGTH – invalid length of the block of words for BC;

BC_BAD_FUNC – this BC device type does not support the called function.

RT_BAD_PAGE – invalid DPRAM page of the RT;

RT_BAD_LENGTH – invalid length of the block of words for RT;

RT_BAD_ADDRESS – invalid RT address;

RT_BAD_FUNC – this RT device type does not support the called function.

**************************************************************************

Interrupt Types

**************************************************************************

BC Interrupts

bcIntNorm – interrupt on normal completion of single message in BC mode (after bcstart). Data: wResult == 0 (a word of message result is equal to zero).

bcIntExc - interrupt on completion of single message with an exclusive situation in BC mode (after bcstart). Data: wResult (a word of message result), wAW1, wAW2 (status words in the order they are received by BC; in messages with one status word, the second status word has undefined value; if an error is detected during message processing (the situation being detected using the word of message result) both STATUSES have undefined value.

bcIntX – interrupt on message(s) completion (break) in BC mode (after bcstartx). Data: wResultX (the extended word of message result), wBase (base of break).

bcIntSig - signal interrupt in BC mode (after bcstartx). Data: wBase (current base).

MT Interrupts

mtIntX - interrupt on break in MT mode (after mtstartx). Data: wResultX (the extended word of result), wBase (base of break).

mtIntSig - signal interrupt in MT mode (after mtstartx). Data: wBase (current base).

RT Interrupts

rtIntCmd - interrupt on mode command in RT mode. Data: wCmd (the mode code of mode command with a mask 0x041F, that is, the command itself (5 lower bits) and the “transmission/receiving” bit of the command word of this command).

rtIntErr - interrupt on message error in RT mode. Data: wStatus (a word of a state of the RT with the set bit “Message Error” and the bits of command word while processing which the error occurred).

rtIntData – interrupt on data transfer command in RT mode. Data: wStatus (a word of a state of the RT, containing 11 lower bits of the COMMAND).

**************************************************************************

Data Types

**************************************************************************

TTmkEventData [Win, Linux]

typedef struct

{

int nInt;

unsigned short wMode;

union

{

struct

{

unsigned short wResult;

unsigned short wAW1;

unsigned short wAW2;

} bc;

struct

{

unsigned short wBase;

unsigned short wResultX;

} bcx;

struct

{

unsigned short wStatus;

unsigned short wCmd;

} rt;

struct

{

unsigned short wBase;

unsigned short wResultX;

} mt;

struct

{

unsigned short wStatus;

} mrt;

};

} TTmkEventData;

The driver transfers to the process the information about every interrupt using the structure of TTmkEventData type. To retrieve the information, the tmkgetevd function is used.

TMK_DATA [Win, Linux]

#define TMK_DATA unsigned short

16 bit unsigned is the data type used to define the driver function parameters.

**************************************************************************

Bus Controller Functions

**************************************************************************

bcdefbase

int bcdefbase(unsigned bcBasePC);

Parameters:

bcBasePC – set base.

Returned value:

0 – function has been successfully completed;

BC_BAD_BASE – specified base value is out of allowed range.

Actions to be completed:

The function customizes the selected BC and the driver to further operation with the DPRAM in the base specified. Calling the function with the invalid base value, results in the erratic situation with BC_BAD_BASE error code.

bcdefbus

int bcdefbus(unsigned bcBus);

Parameters:

bcBus – selected BUS (BUS_1 or BUS_A for the main BUS, BUS_2 or BUS_B for the redundant BUS).

Returned value:

0 - function has been successfully completed;

BC_BAD_BUS – if the redundant BUS is selected for the nonredundant device.

Actions to be completed:

The function customizes the selected BC for the data message via the selected BUS. Further messages are sent via this BUS until bcdefbus or bcreset is called next time (bcreset sets the main BUS). Calling the function with BUS_2 or BUS_B value for the nonredundant device results in the erratic situation with BC_BAD_BUS error code.

bcdefintexc [DOS]

void bcdefintexc(void (far* UserExcBC)(unsigned, unsigned, unsigned));

Parameters:

UserExcBC – defined user’s function called upon the completion of message, started by bcstart function, with an exclusive situation (interrupt bcIntExc); the function must be defined as the far-function returning void type and receiving via its parameters the following three words: a word of message result, first STATUS, second STATUS (the order of the status words is defined by the order the BC receives them); in messages with one status word, the second status word has undefined value; if an error is detected during data message (the error being detected using the word of message result) both STATUSES have undefined value.

Returned value:

N/A

Actions to be completed:

The function defines for the driver the name of the defined user’s function which will be called each time the message over BUS has been completed by the selected BC with the exclusive situation (errors or set status bits have been detected) (See The programmer's guide for more details), if the message was started by bcstart function.

bcdefintnorm [DOS]

void bcdefintnorm(void (far* UserNormBC)(unsigned, unsigned, unsigned));

Parameters:

UserNormBC – defined user’s function called upon the normal completion of the message started by bcstart function (bcIntNorm interrupt); the function must be defined as the far-function returning void type and receiving via its parameters the following three words: the word of message result always equal to 0, and two dummy parameters used for compatibility with UserExcBC (both first and second STATUSES have undefined values).

Returned value:

N/A

Actions to be completed:

The function defines for the driver the name of the defined user’s function which will be called each time upon the normal message completion over BUS by the selected BC (no errors or set status bits have been detected) (See The programmer's guide for more details), if the message was started by bcstart function.

bcdefintsig [DOS]

void bcdefintsig(void (far* UserSigBC)(unsigned));

Parameters:

UserSigBC – defined user’s function called upon the signal interrupt during BC message started by bcstartx function (bcIntSig interrupt); the function must be defined as the far-function returning void type and receiving via its parameters one word: the number of base where the signal interrupt occurred.

Returned value:

N/A

Actions to be completed:

The function defines for the driver the name of the defined user’s function which will be called each time the signal interrupt occurs during BC message started by bcstartx function. If the execution time of the defined user’s function processing signal interrupts exceeds the time interval when the interrupts occur, the defined user's function calls may be skipped for some intermediate interrupts. Defined user’s function must recognize such a situation upon the parameter received (interrupt base number).

bcdefintx [DOS]

void bcdefintx(void (far* UserXBC)(unsigned, unsigned));

Parameters:

UserXBC - defined user’s function called upon the completion of BC message (message chain), started by bcstartx function (bcIntX interrupt); the function must be defined as the far-function returning void type and receiving via its parameters the following two words: the number of base where the break occurred, the word of extended message result.

Returned value:

N/A

Actions to be completed:

The function defines for the driver the name of the defined user’s function which will be called each time the message (message chain) has been completed by the selected BC, if the message (message chain) was started by bcstartx function.

bcdefirqmode

int bcdefirqmode(unsigned bcIrqMode);

Parameters:

bcIrqMode – bit mask of the BC interrupt control mode:

TMK_IRQ_OFF – setting this bit disables the TMKX device interrupt on ISA bus;

BC_GENER1_BL – setting this bit blocks the interrupts upon detection of generation in the main BUS;

BC_GENER2_BL – setting this bit blocks the interrupts upon detection of generation in the redundant BUS.

Returned value:

0 - function has been successfully completed;

BC_BAD_FUNC – selected BC does not support the program control over BC interrupt mode.

Actions to be completed:

The function defines the interrupt mode for the selected BC. By default, bits BC_GENER1_BL and BC_GENER2_BL are always set, as the generation tag may occur asynchronously, but the driver has no ability to process tags. The only relevant response to the generation tag is the device reset upon calling bcreset function or blocking of the generation tags in the user’s interrupt function. If the generation tag bits are not analyzed in the user’s interrupt functions, disabling of blocking interrupts upon generation is prohibited.

bcdeflink

int bcdeflink(unsigned bcBase, unsigned bcCtrlCodeX);

Parameters:

bcBase – base following the currently selected bcdefbase in the chain;

bcCtrlCodeX – extended control code for bcBase base, ORing the message format:

DATA_BC_RT – data transfer BC-RT,

DATA_BC_RT_BRCST - data transfer BC-RT in the broadcast mode,

DATA_RT_BC - data transfer RT-BC,

DATA_RT_RT - data transfer RT-RT,

DATA_RT_RT_BRCST - data transfer RT-RT in the broadcast mode,

CTRL_C_A - mode command of the format COMMAND-STATUS,

CTRL_C_BRCST - mode command of the format COMMAND in the broadcast mode,

CTRL_CD_A - mode command of the format COMMAND+DATA-STATUS,

CTRL_CD_BRCST - mode command of the format COMMAND+DATA in the broadcast mode,

CTRL_C_AD - mode command of the format COMMAND-STATUS+DATA;

Continuation tag:

CX_CONT – upon message completion jump to the next chain base, if no errors or set bits have been detected in the STATUS

CX_STOP – stop upon message completion;

BUS number:

CX_BUS_A (или CX_BUS_0) – main BUS,

CX_BUS_B (или CX_BUS_1) – redundant BUS;

Requirement to generate signal interrupt:

CX_SIG – generate signal interrupt upon message start,

CX_NOSIG – do not generate signal interrupt.

Returned value:

0 - function has been successfully completed;

BC_BAD_BASE – specified base value is out of allowed range;

BC_BAD_FUNC – selected BC does not support message start with bcstartx and chain generation.

Actions to be completed:

The function adds the bcBase of the selected BC to the chain after the currently selected base. If then, when starting bcstartx the control follows the chain up to the current base, and the extended control code for the current base contains the continuation tag CX_CONT, the next message in the chain will be the message from bcBase base with the bcCtrlCodeX control code. The chain will stop if the message with the CX_STOP tag in the control code is completed; the chain will also stop upon any error or set bit in the received STATUS. Calling the function with the invalid base value results in the erratic situation with BC_BAD_BASE error code. This function works on TMKX and TMKXI devices only.

bcgetansw

unsigned long bcgetansw(unsigned bcCtrlCode);

Parameters:

bcCtrlCode – control code defining the message format:

DATA_BC_RT - data transfer BC-RT,

DATA_BC_RT_BRCST - data transfer BC-RT in the broadcast mode,

DATA_RT_BC - data transfer RT-BC,

DATA_RT_RT - data transfer RT-RT,

DATA_RT_RT_BRCST - data transfer RT-RT in the broadcast mode,

CTRL_C_A - mode command of the format COMMAND-STATUS,

CTRL_C_BRCST - mode command of the format COMMAND in the broadcast mode,

CTRL_CD_A - mode command of the format COMMAND+DATA-STATUS,

CTRL_CD_BRCST - mode command of the format COMMAND+DATA in the broadcast mode,

CTRL_C_AD - mode command of the format COMMAND-STATUS+DATA.

Returned value:

Double word:

lower 16 bits - first STATUS in current base,

higher 16 bits – second STATUS in current base.

Actions to be completed:

The function allows receiving STATUSES (two STATUSES for RT-RT format) from the current base, according to the message format defined by the parameter and the COMMAND in word 0  of the current base. Calling the function makes sense if a message has been completed in the current base and the message result code assumes due and possible STATUS. If the STATUS has not been received, the returned value is undefined. If there is no first and/or second STATUS in the specified message code 0xFFFF is returned instead.

bcgetbase

unsigned bcgetbase();

Parameters:

N/A

Returned value:

Current base number.

Actions to be completed:

The function returns the number of the base in the selected BC earlier set by bcdefbase function.

bcgetblk

void bcgetblk(unsigned bcAddr, void far *pcBuffer, unsigned cwLength);

Parameters:

bcAddr – initial address in the base (0 - 63);

pcBuffer – word array address in PC RAM;

cwLength – array length in words (0 - 64).

Returned value:

N/A

Actions to be completed:

The function reads the specified number of words from the current DPRAM base of the selected BC, starting with the bcAddr address, to PC RAM at pcBuffer address. Specifying the invalid array length results in the erratic situation with BC_BAD_LENGTH error code.

bcgetbus

unsigned bcgetbus();

Parameters:

N/A

Returned value:

Currently selected BUS number.

Actions to be completed:

The function returns the BUS number earlier set with the bcdefbus function.

bcgetirqmode

unsigned bcgetirqmode();

Parameters:

N/A

Returned value:

Current values of the BC interrupt mode bits.

Actions to be completed:

Function returns the values of the BC interrupt mode bits earlier set with the bcdefirqmode function.

bcgetlink

unsigned long bcgetlink();

Parameters:

N/A

Returned value:

Double word:

lower 16 bits – next base in the chain following the currently selected base,

higher 16 bits – extended control code for the next base.

Actions to be completed:

For the currently selected base, the function allows retrieving the chain data, earlier set with the bcdeflink function.

bcgetmaxbase

unsigned bcgetmaxbase();

Parameters:

N/A

Returned value:

Maximum base number.

Actions to be completed:

The function returns the maximum allowed number of the DPRAM base in the selected BC, the number can further be used in bcdefbase, bcstart, bcstartx, bcdeflink functions.

bcgetstate

unsigned long bcgetstate();

Parameters:

N/A

Returned value:

Double word:

lower 16 bits – base active in the current message via BUS,

higher 16 bits – base address register contents (state).

Actions to be completed:

The function allows finding out in which of the bases the message chain is processed when the function is called. The function is available on TMKX and TMKXI devices only. Calling the function for other devices results in the erratic situation with BC_BAD_FUNC error code.

bcgetw

unsigned bcgetw(unsigned bcAddr);

Parameters:

bcAddr – address in the base (0 - 63).

Returned value:

Word from the current DPRAM base.

Actions to be completed:

The function reads one word from the current DPRAM base of the selected BC at bcAddr address and returns this word as the result.

bcputblk

void bcputblk(unsigned bcAddr, void far *pcBuffer, unsigned cwLength);

Parameters:

bcAddr – initial address in the base (0 - 63);

pcBuffer – word array address in PC RAM;

cwLength – array length in words (0 - 64).

Returned value:

N/A

Actions to be completed:

Function writes the specified number of words to the current DPRAM base of the selected BC, starting with the bcAddr address, from the PC RAM pcBuffer address. Specifying the invalid array length results in the erratic situation with BC_BAD_LENGTH error code.

bcputw

void bcputw(unsigned bcAddr, unsigned bcData);

Parameters:

bcAddr – address in the base (0 - 63);

bcData – word being written.

Returned value:

N/A

Actions to be completed:

The function writes bcData word to the current DPRAM base of the selected BC at bcAddr address.

bcreset

int bcreset();

Parameters:

N/A

Returned value:

0 – function has been successfully completed,

TMK_BAD_FUNC – the device does not support operation in the BC mode.

Actions to be completed:

The function initializes the selected device switching it to the BC mode. The main BUS is selected on the redundant devices. Current base value is undefined after bcreset function calling.

bcrestore [DOS]

void bcrestore();

Parameters:

N/A

Returned value:

N/A

Actions to be completed:

The function should be used only at the end of the user’s message completion functions if it is necessary to restore the driver state saved during interrupt and the state of the BC that caused interrupt. The function restores the selected device number which was set at the moment of interrupt and the base number of the BC that caused interrupt.

bcstart

int bcstart(unsigned bcBase, unsigned bcCtrlCode);

Parameters:

bcBase – base from which the data message is started;

bcCtrlCode – control code defining the message format:

DATA_BC_RT - data transfer BC-RT,

DATA_BC_RT_BRCST - data transfer BC-RT in the broadcast mode,

DATA_RT_BC - data transfer RT-BC,

DATA_RT_RT - data transfer RT-RT,

DATA_RT_RT_BRCST - data transfer RT-RT in the broadcast mode,

CTRL_C_A - mode command of the format COMMAND-STATUS,

CTRL_C_BRCST - mode command of the format COMMAND in the broadcast mode,

CTRL_CD_A - mode command of the format COMMAND+DATA-STATUS,

CTRL_CD_BRCST - mode command of the format COMMAND+DATA in the broadcast mode,

CTRL_C_AD - mode command of the format COMMAND-STATUS+DATA.

Returned value:

0 – function has been successfully completed;

BC_BAD_BASE – specified base value is out of allowed range.

Actions to be completed:

The functions initiates the message start via BUS earlier set by calling bcdefbus function, from DPRAM base bcBase of the selected BC in the format defined by bcCtrlCode and command words of the message earlier written in this base. Calling the function with the invalid base value results in the erratic situation with BC_BAD_BASE error code. This function works on all devices and allows starting only a single message from the specified base. Upon message completion the normal completion code is generated and the user’s message completion function is called. The function set with bcdefintnorm or bcdefinexc.

bcstartx

int bcstartx(unsigned bcBase, unsigned bcCtrlCodeX);

Parameters:

bcBase - base from which the data message is started;

bcCtrlCodeX – extended control code, ORing the message format:

DATA_BC_RT - data transfer BC-RT,

DATA_BC_RT_BRCST - data transfer BC-RT in the broadcast mode,

DATA_RT_BC - data transfer RT-BC,

DATA_RT_RT - data transfer RT-RT,

DATA_RT_RT_BRCST - data transfer RT-RT in the broadcast mode,

CTRL_C_A - mode command of the format COMMAND-STATUS,

CTRL_C_BRCST - mode command of the format COMMAND in the broadcast mode,

CTRL_CD_A - mode command of the format COMMAND+DATA-STATUS,

CTRL_CD_BRCST - mode command of the format COMMAND+DATA in the broadcast mode,

CTRL_C_AD - mode command of the format COMMAND-STATUS+DATA;

Continuation tag:

CX_CONT - upon message completion jump to the next chain base, if no errors or set bits have been detected in the STATUS,

CX_STOP - stop upon message completion;

BUS number:

CX_BUS_A (или CX_BUS_0) – main BUS,

CX_BUS_B (или CX_BUS_1) – redundant BUS;

Requirement to generate signal interrupt:

CX_SIG - generate signal interrupt upon message start,

CX_NOSIG - do not generate signal interrupt.

Returned value:

0 – function has been successfully completed;

BC_BAD_BASE – specified base value if out of allowed range;

BC_BAD_FUNC – selected BC does not support message start with bcstartx.

Actions to be completed:

The function initiates the message start via BUS set by bcCtrlCodeX from DPRAM base of the selected BC in the format set by bcCtrlCodeX and command words of the message previously written in this base. Calling the function with the invalid base value results in the erratic situation with BC_BAD_BASE error code. This function works on TMKX and TMKXI devices only and allows starting a single message from the specified base as well as the message chain started with the specified base. Chain start is set by adding CX_CONT to  bcCtrlCodeX. The chain must be generated by calling bcdeflink. Upon message completion the extended message completion code is generated, and the user’s message completion function set by calling bcdeflink is called. If set signal interrupt bits are found in the control words while processing the chain, the user’s signal interrupt processing function set by bcdefintsig will be called.

bcstop

int bcstop();

Parameters:

N/A

Returned value:

0 – function has been successfully completed;

BC_BAD_FUNC – selected BC does not support message start with bcstartx and message stop with bcstop.

Actions to be completed:

The function sets the chain continuation tag of the current BC to zero, that results in the chain stop after the current chain message started by calling bcstartx has been completed. The function works on TMKX and TMKXI devices only.

**************************************************************************

Monitor Functions (TMKX and TMKXI Devices)

**************************************************************************

mtdefbase

int mtdefbase(unsigned mtBasePC);

Parameters:

mtBasePC – base being set.

Returned value:

0 – function has been successfully completed;

MT_BAD_BASE – specified base value is out of allowed range.

Actions to be completed:

The function customizes the selected MT and the driver to further operate with the DPRAM in the specified base. Calling the function with the invalid base value results in the erratic situation with MT_BAD_BASE error code.

mtdefintsig [DOS]

void mtdefintsig(void (far* UserSigMT)(unsigned));

Parameters:

UserSigMT – defined user’s function called when the signal interrupt occurs during operation of the MT started with mtstartx function (mtIntSig interrupt); the function must be defined as the far-function returning void type and receiving via its parameters one word: the number of base where signal interrupt occurred.

Returned value:

N/A

Actions to be completed:

The function defines for the driver the name of the defined user’s function which will be called each time when the signal interrupt occurs while operation of the MT started with mtstartx function. If the execution time of the defined user’s function processing signal interrupts exceeds the time interval when the interrupts occur, the defined user's function calls for some intermediate interrupts may be skipped. Defined user’s function must recognize such a situation upon the parameter received (interrupt base number).

mtdefintx [DOS]

void mtdefintx(void (far* UserXMT)(unsigned, unsigned));

Parameters:

UserXMT – defined user’s function called upon operation completion of the MT started with mtstartx function (mtIntX interrupt); the function must be defined as the far-function returning void type and receiving via its parameters the following two words: the number of the base where the break occurred, the word of extended message result.

Returned value:

N/A

Actions to be completed:

The function defines for the driver the name of the defined user’s function which will be called each time when the selected MT completes operation, if the operation was started with mtstartx function.

mtdefirqmode

int mtdefirqmode(unsigned mtIrqMode);

Parameters:

mtIrqMode - mask of the MT interrupt control mode:

TMK_IRQ_OFF – setting this bit disables the TMKX device interrupt on ISA bus;

MT_GENER1_BL – setting this bit blocks interrupts upon detection of generation in the main BUS;

MT_GENER2_BL – setting this bit blocks interrupts upon detection of generation in the redundant BUS.

Returned value:

0 - the function has been successfully completed;

MT_BAD_FUNC – selected MТ does not support the program control over MT interrupts mode.

Actions to be completed:

The function defines the interrupts mode of the selected MT. By default bits MT_GENER1_BL and MT_GENER2_BL are always set, as the generation tag may occur asynchronously but the driver has no ability to process tags. The only relevant response to the generation tag is the device reset upon calling mtreset function or blocking of the generation tags in the user’s interrupt function. If the generation tag bits are not analyzed in the user’s interrupt functions, disabling of blocking interrupts upon generation is prohibited.

mtdeflink

int mtdeflink(unsigned mtBase, unsigned mtCtrlCodeX);

Parameters:

mtBase - base following the currently selected mtdefbase in the chain;

mtCtrlCodeX - extended control code for mtBase base, ORing the continuation tag:

CX_CONT - upon message completion jump to the next chain base.

CX_STOP - stop upon message completion;

Break (interrupt) tag upon error:

CX_INT – МТ break upon detecting an error or set bit in the STATUS,

CX_NOINT – no MT break upon detecting an error or set bit in the STATUS;

Requirement to generate signal interrupt:

CX_SIG - generate signal interrupt upon message start,

CX_NOSIG - do not generate signal interrupt.

Returned value:

0 - the function has been successfully completed;

MT_BAD_BASE - specified base value is out of allowed range;

MT_BAD_FUNC - selected MТ does not support the message start with mtstartx and chain generation.

Actions to be completed:

The function adds the mtBase of the selected MT to the chain after the currently selected base. If then, when starting mtstartx the control follows the chain up to the current base, and the extended control code for the current base contains the continuation tag CX_CONT, the next message in the chain will be the message from mtBase base with the mtCtrlCodeX control code. The chain will stop if the message with the CX_STOP tag in the control code is completed; the chain will also stop upon any error or set bit in the received STATUS and CX_INT tag is set. Calling the function with the invalid base value results in the erratic situation with MT_BAD_BASE error code.

mtgetsw

unsigned mtgetsw();

Parameters:

N/A

Returned value:

Extended word of message result for the current base.

Actions to be completed:

The function allows to retrieve the extended word of message result from the current base in case when the monitor has written a message to the base, and has not stopped but jumped to the next base in the chain. Upon monitor break the extended word of message result can be retrieved only from parameters of the function set with mtdefintx and called upon MT break interrupt.

mtgetbase

unsigned mtgetbase();

Parameters:

N/A

Returned value:

Current base number.

Actions to be completed:

The function returns the number of the base in the selected MT earlier set by mtdefbase function.

mtgetblk

void mtgetblk(unsigned mtAddr, void far *pcBuffer, unsigned cwLength);

Parameters:

mtAddr - initial address in the base (0 - 63);

pcBuffer - word array address in PC RAM;

cwLength – array length in words (0 - 64).

Returned value:

N/A

Actions to be completed:

The function reads the specified number of words from the current DPRAM base of the selected MT, starting with the mtAddr address, to the PC RAM at pcBuffer address. Specifying the invalid array length results in the erratic situation with MT_BAD_LENGTH error code.

mtgetirqmode

unsigned mtgetirqmode();

Parameters:

N/A

Returned value:

Current value of the MT interrupts mode bits.

Actions to be completed:

The function returns the values of the MT interrupts mode bits earlier set with the mtdefirqmode function.

mtgetlink

unsigned long mtgetlink();

Parameters:

N/A

Returned value:

Double word:

lower 16 bits - next base in the chain following the currently selected base,

higher 16 bits - extended control code for the next base.

Actions to be completed:

For the currently selected base, the function allows retrieving the chain data, earlier set with the mtdeflink function.

mtgetmaxbase

unsigned mtgetmaxbase();

Parameters:

N/A

Returned value:

Maximum base number.

Actions to be completed:

The function returns the maximum allowed number of the DPRAM base in the selected MT, the number can further be used in mtdefbase, mtstartx, mtdeflink functions.

mtgetstate

unsigned long mtgetstate();

Parameters:

N/A

Returned value:

Double word:

lower 16 bits - base active in the current message via BUS,

higher 16 bits - base address register contents (state).

Actions to be completed:

The function allows finding out in which of the bases the monitor message chain is processed when the function is called.

mtgetw

unsigned mtgetw(unsigned mtAddr);

Parameters:

mtAddr – address in the base (0 - 63).

Returned value:

Word from the current DPRAM base.

Actions to be completed:

The function reads one word from the current DPRAM base of the selected MT at mtAddr address and returns it as the result.

mtputblk

void mtputblk(unsigned mtAddr, void far *pcBuffer, unsigned cwLength);

Parameters:

mtAddr – initial address in the base (0 - 63);

pcBuffer - word array address in PC RAM;

cwLength - array length in words  (0 - 64).

Returned value:

N/A

Actions to be completed:

The function writes the specified number of words to the current DPRAM base of the selected MT, starting with the mtAddr address, from the PC RAM at pcBuffer address. Specifying the invalid array length results in the erratic situation with MT_BAD_LENGTH error code.

mtputw

void mtputw(unsigned mtAddr, unsigned mtData);

Parameters:

mtAddr – address in the base (0 - 63);

mtData – word being written.

Returned value:

N/A

Actions to be completed:

The function writes the mtData word to the current DPRAM base of the selected MT at the mtAddr address.

mtreset

int mtreset();

Parameters:

N/A

Returned value:

0 - the function has been successfully completed,

TMK_BAD_FUNC – the device does not support operation in the MT mode.

Actions to be completed:

The function initializes the selected device switching it to the MT mode. Current base value is undefined after mtreset function calling.

mtrestore [DOS]

void mtrestore();

Parameters:

N/A

Returned value:

N/A

Actions to be completed:

The function should be used only at the end of the user’s message completion functions if it is necessary to restore the driver state saved during interrupt and the state of the MT that caused interrupt. The function restores the selected device number which was set at the moment of interrupt and the base number of the MT that caused interrupt.

mtstartx

int mtstartx(unsigned mtBase, unsigned mtCtrlCodeX);

Parameters:

mtBase – base from which the data message is started;

mtCtrlCodeX - extended control code for mtBase base, ORing the continuation tag:

CX_CONT - upon message completion jump to the next chain base,

CX_STOP - stop upon message completion;

Break (interrupt) tag upon error:

CX_INT - МТ break upon detecting an error or set bit in the STATUS,

CX_NOINT - no MT break upon detecting an error or set bit in the STATUS;

Requirement to generate signal interrupt:

CX_SIG - generate signal interrupt upon message start,

CX_NOSIG - do not generate signal interrupt.

Returned value:

0 - the function has been successfully completed;

MT_BAD_BASE - specified base value is out of allowed range;

MT_BAD_FUNC – selected MT does not support message start with mtstartx.

Actions to be completed:

The function initiates the BUS start from the DPRAM mtBase of selected MT in the format with the first base tags set with mtCtrlCodeX. Calling the function with the invalid base value results in the erratic situation with MT_BAD_BASE error code. This function allows to start waiting for a single message as well as waiting for several messages in the message chain started with the specified base. Chain start is set by adding CX_CONT to mtCtrlCodeX. The chain must be generated by calling mtdeflink. Upon message completion the extended message completion code is generated, and the user’s message completion function set by calling mtdeflink is called. If the signal interrupt set bits are found in the control words while processing the chain, the user’s signal interrupt processing function set by mtdefintsig will be called.

mtstop

int mtstop();

Parameters:

N/A

Returned value:

0 - the function has been successfully completed;

MT_BAD_FUNC – selected MT does not support message start with mtstartx and break with mtstop.

Actions to be completed:

The function sets the chain continuation tag of the current selected MT to zero, that results in the chain stop after the current chain message started by calling mtstartx has been completed. This function usage makes sense only if the messages are delivered to the BUS continuously. If there no message in the channel, there is no use to call mtstop, as the monitor will wait for at least one message upon receiving which it will stop. In general, the monitor can be stopped by calling mtreset. Upon calling the function the MT will be reset immediately and will stop the message receiving and will not generate any interrupts upon chain completion.

**************************************************************************

Remote Terminal Functions

**************************************************************************

rtbusy

unsigned rtbusy();

Parameters:

N/A

Returned value:

0 – selected subaddress is free;

1 – selected subaddress is busy in BUS data message.

Actions to be completed:

The function determines the availability of the current subaddress of the selected RT in the mode without flags.

rtclranswbits

void rtclranswbits(unsigned rtClrBits);

Parameters:

rtClrBits – mask setting the specified bits of the STATUS to 0:

SREQ - bit "Service Request",

BUSY - bit "Subsystem Busy",

SSFL - bit "Subsystem Flag",

RTFL - bit "Terminal Flag",

DNBA - bit "Ready for Dynamic Bus Control Acceptance".

Returned value:

N/A

Actions to be completed:

The function resets the set bits of the status words returned by the selected RT.

rtclrflag

void rtclrflag();

Parameters:

N/A

Returned value:

N/A

Actions to be completed:

The function resets the flag in the flag word of the current subaddress of the selected RT in the RT mode with flags.

rtdefaddress

int rtdefaddress(unsigned rtAddress);

Parameters:

rtAddress – RT address in the BUS (0 - 30).

Returned value:

0 - the function has been successfully completed;

RT_BAD_ADDRESS – invalid RT address is specified;

RT_BAD_FUNC – selected RT does not support the program setting of the RT address in the BUS.

Actions to be completed:

The function programs the address of the selected RT in the BUS. If the device type of the selected RT does not support address programming (the address is set by jumpers on the device), an erratic situation with the RT_BAD_FUNC error code occurs. Specifying the invalid RT address results in the erratic situation with the RT_BAD_ADDRESS error code.

rtdefintcmd [DOS]

void rtdefintcmd(void (far* UserCmdRT)(unsigned));

Parameters:

UserCmdRT – defined user’s function called when mode command is received (rtIntCmd interrupt); the function must be defined as the far-function returning void type and receiving via its parameter one word: command code containing the command itself (lower 5 bits) and the bit “receiving/trasmitting” of the command word of this command.

Returned value:

N/A

Actions to be completed:

The function defines for the driver the name of the defined user’s function which will be called each time the mode command is received by the selected RT, while the command cannot be completed by the device itself (See The programmer's guide).

rtdefintdata [DOS]

void rtdefintdata(void (far* UserDataRT)(unsigned));

Parameters:

UserDataRT – defined user’s function called after the RT completed the data transfer command (rtIntData interrupt); the function must be defined as the far-function returning void type and receiving via its parameter one word: a word of a state of the RT containing lower 11 bits of the COMMAND.

Returned value:

N/A

Actions to be completed:

The function defines for the driver the name of the defined user’s function which will be called each time when RT completes the data transfer command. Not all devices are able to generate such interrupt by hardware, therefore, the actually specified defined user’s function will be called only on TMKX and TMKXI devices. By default generation of such interrupts is disabled, therefore in order to generate them, they should be enabled by calling rtdefirqmode(rtgetirqmode()&~RT_DATA_BL).

rtdefinterr [DOS]

void rtdefinterr(void (far* UserErrRT)(unsigned));

Parameters:

UserErrRT - defined user’s function called upon detecting the BUS error during data message between the selected RT and the channel (rtIntErr interrupt); the function must be defined as the far-function returning void type and receiving via its parameter one word: a word of a state of the RT with the bit "Message Error" set and the bits of the command word while processing which the error occurred.

Returned value:

N/A

Actions to be completed:

The function defines for the driver the name of the defined user’s function which will be called each time when the BUS data message error occurs in the selected RT (See The programmer’s guide).

rtdefirqmode

int rtdefirqmode(unsigned rtIrqMode);

Parameters:

rtIrqMode – bit mask of the RT interrupt control mode:

TMK_IRQ_OFF - setting this bit disables the TMKX device interrupt on ISA bus;

RT_GENER1_BL – setting this bit disables the interrupts upon detection of generation in the main BUS;

RT_GENER2_BL - setting this bit disables the interrupts upon detection of generation in the redundant BUS;

RT_DATA_BL - setting this bit disables the interrupts upon data transfer commands.

Returned value:

0 - the function has been successfully completed;

RT_BAD_FUNC – selected RT does not support the program control over RT interrupt mode.

Actions to be completed:

The function defines the interrupts mode of the selected RT. By default bits RT_GENER1_BL and RT_GENER2_BL are always set, as the generation tag may occur asynchronously but the driver has no ability to process tags. The only relevant response to the generation tag is the device reset upon calling rtreset function or blocking of the generation tags in the user’s interrupt function. If the generation tag bits are not analyzed in the user’s interrupt functions, disabling of blocking interrupts upon generation is prohibited. Bit RT_DATA_BL is also set by default; therefore the RT does not generate any interrupts upon data transfer commands. To enable these interrupts, the rtdefirqmode(rtgetirqmode()&~RT_DATA_BL) should be called.

rtgetirqmode

unsigned rtgetirqmode();

Parameters:

N/A

Returned value:

Current values of the RT interrupt mode bits.

Actions to be completed:

The function returns the values of the RT interrupt mode bits earlier set with the rtdefirqmode function.

rtdefmode

int rtdefmode(unsigned rtMode);

Parameters:

rtMode – RT mode set by the ORing (combining by “OR”) of the following mode bits:

RT_HBIT_MODE – enables the RT mode using instrumentation bit in the command word subaddress field (by default the mode is enabled);

RT_FLAG_MODE – enables the RT mode with flags (by default the mode is disabled);

RT_BRCST_MODE – enables the RT mode when the command words with the broadcast address are recognized (by default the mode is enabled).

Returned value:

0 - the function has been successfully completed;

RT_BAD_FUNC – selected RT does not support the programmable RT operation modes.

Actions to be completed:

The function programs the operation modes of the selected RT; rtreset function called directly after calling tmkconfig, switches the RT to (RT_HBIT_MODE | RT_BRCST_MODE), the subsequent calls of rtreset do not change the current RT modes; TMK400, TMKMPC devices do not allow programmable control over RT modes, for these devices the RT modes are defined by setting jumpers on the device. Calling the function for these device types results in the erratic situation with the RT_BAD_FUNC error code; full control over all modes is allowed by RTMK400 device; TMKX and TMKXI devices allow controlling the RT_FLAG_MODE and RT_BRCST_MODE.

rtdefpage

int rtdefpage(unsigned rtPage);

Parameters:

rtPage – selected DPRAM page.

Returned value:

0 - the function has been successfully completed;

RT_BAD_PAGE – invalid DPRAM page number is specified.

Actions to be completed:

The function sets the rtPage page as the current page in the selected RT. Calling the function with the invalid page number results in the erratic situation with RT_BAD_PAGE error code.

rtdefpagebus

int rtdefpagebus(unsigned rtPageBus);

Parameters:

rtPageBus – DPRAM page accessible for the BUS.

Returned value:

0 - the function has been successfully completed;

RT_BAD_PAGE – invalid DPRAM page number is specified;

RT_BAD_FUNC – selected RT does not support separate definition of the page accessible on the side of the BUS and the page accessible on the side of the PC.

Actions to be completed:

The function makes the rtPageBus page in the selected RT accessible for the BUS. This does not affect the page accessible for the PC. Calling the function with the page number invalid for the selected RT results in the erratic situation with RT_BAD_PAGE error code. If the device type of the selected RT does not support separate definition of the page accessible on the side of the BUS and the page accessible on the side of the PC, an erratic situation with RT_BAD_FUNC error code occurs.

rtdefpagepc

int rtdefpagepc(unsigned rtPagePC);

Parameters:

rtPagePC – DPRAM page accessible for the PC.

Returned value:

0 - the function has been successfully completed;

RT_BAD_PAGE – invalid DPRAM page number is specified;

RT_BAD_FUNC - selected RT does not support separate definition of the page accessible on the side of the BUS and the page accessible on the side of the PC.

Actions to be completed:

The function makes the rtPageBus page in the selected RT accessible for the PC. This does not affect the page accessible for the BUS. Calling the function with the page number invalid for the selected RT results in the erratic situation with RT_BAD_PAGE error code. If the device type of the selected RT does not support separate definition of the page accessible on the side of the BUS and the page accessible on the side of the PC, an erratic situation with RT_BAD_FUNC error code occurs.

rtdefsubaddr

void rtdefsubaddr(unsigned rtDir, unsigned rtSubAddr);

Parameters:

rtDir – selecting subaddress of receiving or transmission:

RT_RECEIVE – subaddress of receiving,

RT_TRANSMIT – subaddress of transmission;

rtSubAddr – subaddress number:

1 - 30 in the mode without instrumentation bit,

16 - 30 in the mode with the instrumentation bit.

Returned value:

N/A

Actions to be completed:

The function customizes the selected RT and the driver for further operation with the DPRAM in the specified subaddress.

rtenable [Win, Linux]

unsigned rtenable(unsigned rtEnable);

Parameters:

rtEnable – defining the function operation:

RT_ENABLE – switching on (enabling) the RT,

RT_DISABLE – switching off (disabling) the RT,

RT_GET_ENABLE – reading the current value of the RT enabled operation;

Returned value (when rtEnable != RT_GET_ENABLE):

0 - the function has been successfully completed;

RT_BAD_FUNC – error while specifying the parameter.

Returned value (when rtEnable == RT_GET_ENABLE):

RT_ENABLE – RT is switched on,

RT_DISABLE – RT is switched off.

Actions to be completed:

The function allows switching on/off the selected RT without disabling the RT mode itself. The function is mainly used on the devices operating in the multi-address RT mode, that is, it controls the virtual RTs within one physical device. Besides that, this function can be used on the devices of RTMK400, TMKX, TMKXI types in the RT mode. The devices of TMK400 and TMKMPC types do not have the hardware required for proper operation of this function.

rtgetaddress

unsigned rtgetaddress();

Parameters:

N/A

Returned value:

RT address in the BUS.

Actions to be completed:

The function reads an address of the selected RT in the BUS from the RT itself (if the type of the device used allows that) or from the internal driver variable.  If the type of the selected RT does not support address programming (the address is defined by setting jumpers on the device) and does not support program definition of the address set, an erratic situation with RT_BAD_FUNC error code occurs.

rtgetanswbits

unsigned rtgetanswbits();

Parameters:

N/A

Returned value:

State of bits of the flag field in STATUS:

SREQ - bit "Service Request",

BUSY - bit "Subsystem Busy",

SSFL - bit "Subsystem Flag",

RTFL - bit "Terminal Flag",

DNBA - bit "Ready to Dynamic Bus Control Acceptance".

Actions to be completed:

The function returns the current state of the bits setting the values of the corresponding bits of the flag field in the STATUS of the selected RT, the values having been set earlier with rtsetanswbits and rtclranswbits functions.

rtgetblk

void rtgetblk(unsigned rtAddr, void far *pcBuffer, unsigned cwLength);

Parameters:

rtAddr – initial address in the subaddress (0 - 31);

pcBuffer - word array address in PC RAM;

cwLength - array length in words  (0 - 32).

Returned value:

N/A

Actions to be completed:

The function reads the specified number of words from the current DPRAM subaddress of the selected RT, starting with the rtAddr address, to the PC RAM at pcBuffer address. Specifying the invalid array length results in the erratic situation with RT_BAD_LENGTH error code.

rtgetcmddata

unsigned rtgetcmddata(unsigned rtBusCommand);

Parameters:

rtBusCommand – mode command with DATA.

Returned value:

DATA for the specified command.

Actions to be completed:

The function reads the DATA for the specified mode command (the command code and the bit “receiving/transmission” must be set). NOTE! After this command is called the current address value is undefined.

rtgetflag

unsigned rtgetflag(unsigned rtDir, unsigned rtSubAddr);

Parameters:

rtDir - selecting subaddress of receiving or transmission:

RT_RECEIVE - subaddress of receiving,

RT_TRANSMIT - subaddress of transmission;

rtSubAddr - subaddress number:

1 - 30 in the mode without instrumentation bit,

16 - 30 in the mode with instrumentation bit.

Returned value:

Flag word for the specified subaddress.

Actions to be completed:

The function reads the flag word for the specified subaddress in the selected RT and customizes the selected RT and the driver for further operation with the DPRAM in this subaddress.

rtgetflags

void rtgetflags(void far *pcBuffer, unsigned rtDir, unsigned rtFlagMin, unsigned rtFlagMax);

Parameters:

pcBuffer – address of the array used for reading flag words block in the PC RAM;

rtDir - selecting subaddresses of receiving or transmission:

RT_RECEIVE - subaddresses of receiving,

RT_TRANSMIT - subaddresses of transmission;

rtFlagMin, rtFlagMax – range of subaddresses for which the flag words must be read (1 - 30).

Returned value:

N/A

Actions to be completed:

The function reads from the DPRAM the block of the flag words for the specified subaddresses range of the selected RT. When rtFlagMin==rtFlagMax, one flag word is read. When rtFlagMin>rtFlagMax, the function does not read anything. NOTE! After this command is called the current subaddress value is undefined.

rtgetmaxpage

unsigned rtgetmaxpage();

Parameters:

N/A

Returned value:

Maximum page number.

Actions to be completed:

The function returns the maximum allowed number of the DPRAM page in the selected RT, the page number may be used in rtdefpage, rtdefpagebus and rtdefpagepc functions.

rtgetmode

unsigned rtgetmode();

Parameters:

N/A

Returned value:

The current mode of the RT operation, displayed by the combination of the set bits of the mode:

RT_HBIT_MODE – the RT operation mode with the usage of the instrumentation bit in the subaddress field of the command word is switched on;

RT_FLAG_MODE – RT mode with flags is switched on;

RT_BRCST_MODE – RT mode when the command words with the broadcast address are recognized is switched on;

For the device types that do not support the programmable RT operation modes 0 is always returned.

Actions to be completed:

The function returns the current modes of the selected RT for the RTMK400, TMKX, TMKXI devices; rtreset function called directly after calling tmkconfig, switches the RT to (RT_HBIT_MODE | RT_BRCST_MODE), subsequent calls of rtreset do not change the current RT modes; TMK400, TMKMPC devices do not allow the programmable RT operation modes, the RT modes on these devices are defined by setting jumpers on the devices, calling the function for these device types results in the erratic situation with RT_BAD_FUNC error code.

rtgetpage

unsigned rtgetpage();

Parameters:

N/A

Returned value:

Current page number.

Actions to be completed:

The function returns the page number in the selected RT, the page number having been earlier specified with rtdefpage function.

rtgetpagebus

unsigned rtgetpagebus();

Parameters:

N/A

Returned value:

Current number of the page accessible for the BUS.

Actions to be completed:

The function returns the number of the page accessible for the BUS in the selected RT, the page number having been set earlier with rtdefpagebus function.

rtgetpagepc

unsigned rtgetpagepc();

Parameters:

N/A

Returned value:

Current number of the page accessible for the PC.

Actions to be completed:

The function returns the number of the page accessible for the PC in the selected RT, the page number having been set earlier with rtdefpagepc function.

rtgetstate

unsigned rtgetstate();

Parameters:

N/A

Returned value:

Word of state of the RT.

Actions to be completed:

The function reads the word of state of the selected RT, the word containing the bits of the last COMMAND received by the RT, bit of usage of a subaddress, message error bit and subsystem data exchange error bit (only for TMK400, RTMK400, TMKMPC devices).

rtgetsubaddr

unsigned rtgetsubaddr();

Parameters:

N/A

Returned value:

Number of the current subaddress with the bit “receiving/transmitting”.

Actions to be completed:

The function returns the number of the current subaddress of the selected RT, the subaddress having been set earlier with rtdefsubaddr, rtlock, rtgetflag functions.

rtgetw

unsigned rtgetw(unsigned rtAddr);

Parameters:

rtAddr – address in subaddress (0 - 31).

Returned value:

Word from the current DPRAM subaddress.

Actions to be completed:

The function reads one word from the current DPRAM subaddress of the selected RT at the rtAddr address and returns it as the result.

rtlock

void rtlock(unsigned rtDir, unsigned rtSubAddr);

Parameters:

rtDir - selecting subaddress of receiving or transmission:

RT_RECEIVE - subaddress of receiving,

RT_TRANSMIT - subaddress of transmission;

rtSubAddr - subaddress number:

1 - 30 in the mode without instrumentation bit,

16 - 30 in the mode with instrumentation bit.

Returned value:

N/A

Actions to be completed:

The function customizes the selected RT and the driver to further operate with the DPRAM in the specified subaddress and simultaneously disables the subaddress used by the BUS to access DPRAM. If at the moment the function was called the BUS was exchanging messages with this subaddress, disabling will be delayed until the message is completed. Disabling success can be verified by subsequently calling rtbusy.

rtputblk

void rtputblk(unsigned rtAddr, void far *pcBuffer, unsigned cwLength);

Parameters:

rtAddr – initial address in the subaddress (0 - 31);

pcBuffer - word array address in PC RAM;

cwLength - array length in words  (0 - 32).

Returned value:

N/A

Actions to be completed:

The function writes the specified number of words to the current DPRAM subaddress of the selected RT, starting with rtAddr address, from the PC RAM at pcBuffer address. Specifying the invalid array length results in the erratic situation with RT_BAD_LENGTH error code.

rtputcmddata

void rtputcmddata(unsigned rtBusCommand, unsigned rtData);

Parameters:

rtBusCommand – mode command with DATA;

rtData – DATA for the specified command.

Returned value:

N/A

Actions to be completed:

The function writes the DATA for the specified mode command (the command code and the bit “receivubg/transmitting” must be set). NOTE! After this command is called the current subaddress value is undefined.

rtputflags

void rtputflags(void far *pcBuffer, unsigned rtDir, unsigned rtFlagMin, unsigned rtFlagMax);

Parameters:

pcBuffer – array address in the PC RAM with the block of flag words;

rtDir - selecting subaddress of receiving or transmission:

RT_RECEIVE – subaddresses of receiving,

RT_TRANSMIT – subaddresses of transmission;

rtFlagMin, rtFlagMax - of subaddresses for which the flag words must be written (1 - 30).

Returned value:

N/A

Actions to be completed:

The function writes the block of flag words for the specified subaddresses range of the selected RT to the DPRAM. When rtFlagMin==rtFlagMax, one flag word is written. When rtFlagMin>rtFlagMax, the function does not write anything. NOTE! After this command is called the current subaddress value is undefined.

rtputw

void rtputw(unsigned rtAddr, unsigned rtData);

Parameters:

rtAddr – address in the subaddress (0 - 31);

rtData – the word being written.

Returned value:

N/A

Actions to be completed:

The function writes the rtData word to the current DPRAM subaddress of the selected RT at the rtAddr address.

rtreset

int rtreset();

Parameters:

N/A

Returned value:

0 - the function has been successfully completed,

TMK_BAD_FUNC – the device does not support the operation in the RT mode.

Actions to be completed:

The function initializes the selected device switching it to the RT mode. The zero page of the DPRAM is selected and all the STATUS bits are reset. To set the bits, rtsetanswbits function must be used. The value of the current subaddress after calling rtreset is undefined. If the device allows programming the RT operation modes, the first rtreset call performed after tmkconfig call switches the RT to the mode with instrumentation bit and the mode of receiving broadcast messages and switches off the mode with flags. Subsequent calls of the rtreset function do not change the RT modes set by the moment of the call.

rtrestore [DOS]

void rtrestore();

Parameters:

N/A

Returned value:

N/A

Actions to be completed:

The function should be used only at the end of the user’s RT interrupt processing functions if it is necessary to restore the driver state saved during interrupt and the state of the RT that caused interrupt. The function restores the saved selected device number which was set at the moment of interrupt and the saved subaddress number of the RT that caused the interrupt.

rtsetanswbits

void rtsetanswbits(unsigned rtSetBits);

Parameters:

rtSetBits – mask setting the specified bits in the STATUS to 1:

SREQ - bit "Service Request",

BUSY - bit "Subsystem Busy",

SSFL - bit "Subsystem Flag",

RTFL - bit "Terminal Flag",

DNBA - bit "Ready to Dynamic Bus Control Acceptance".

Returned value:

N/A

Actions to be completed:

The function sets the set bits in the status words generated by the selected RT.

rtsetflag

void rtsetflag();

Parameters:

N/A

Returned value:

N/A

Actions to be completed:

The function sets the flag in the flag word of the current subaddress of the selected RT in the RT mode with flags.

rtunlock

void rtunlock();

Parameters:

N/A

Returned value:

N/A

Actions to be completed:

The function unlocks the current subaddress of the selected RT in the RT mode without flags, subaddress having been locked earlier by rtlock function.

**************************************************************************

Multi-address RT Functions (TX6/TE6 devices)

**************************************************************************

mrtconfig [Win, Linux]

unsigned long mrtconfig(int mrtNumber);

Parameters:

mrtNumber – the number of the device being configured starting with 0;

Returned value:

lower 16 bits – minimum number of the VRT connected when the function was called,

higher 16 bits – number of VRTs connected when the function was called;

or

0 – error during function execution.

Actions to be completed:

The function connects all VRT located on the physical MRT with the specified address to the calling process. If the device with this number does not exist or has been already connected to another proces, 0 value is returned.

mrtdefbrcsubaddr0 [Win, Linux]

void mrtdefbrcsubaddr0();

Parameters:

N/A

Returned value:

N/A

Actions to be completed:

The function customizes the selected VRT, MRT and the driver for further operation with the DPRAM in the subaddress used for receiving the broadcast messages in the selected MRT.

mrtgetmaxn [Win, Linux]

int mrtgetmaxn();

Parameters:

N/A

Returned value:

Maximum allowed number of the MRT device for this driver.

Actions to be completed:

The function returns the device maximum number which can be set with mrtconfig function. If MRT exists tmkgetmaxn function returns the maximum existing VRT number, as opposed to mrtgetmaxn.

mrtgetstate [Win, Linux]

unsigned mrtgetstate();

Parameters:

N/A

Returned value:

MRT word of state.

Actions to be completed:

The function reads the selected MRT word of state containing the bits of the last COMMAND received by the MRT, the bit of usage of a subaddress, the bit of Message Error in BUS.

mrtreset [Win, Linux]

int mrtreset();

Parameters:

N/A

Returned value:

0 - the function has been successfully completed,

TMK_BAD_FUNC – the device is not an MRT device.

Actions to be completed:

The function initializes the selected MRT device. Calling mrtreset switches on the MRT mode with instrumentation bit and the mode of receiving broadcast messages and switches off the mode with flags.

mrtselected [Win, Linux]

int mrtselected();

Parameters:

N/A

Returned value:

Number of the selected MRT device.

Actions to be completed:

The function returns the number of the MRT containing the VRT earlier selected with tmkconfig or tmkselect functions.

**************************************************************************

General Functions

**************************************************************************

TmkOpen [Win, Linux]

int TmkOpen();

Parameters:

N/A

Returned value:

0 - the function has been successfully completed,

VTMK_BAD_VERSION – incompatible driver version,

STATUS error code has non-zero value.

Actions to be completed:

The function connects the driver to the process which has called the function.

TmkClose [Win, Linux]

void TmkClose();

Parameters:

N/A

Returned value:

N/A

Actions to be completed:

The function disconnects the driver from the process which has called the function.

tmkclrcwbits

void tmkclrcwbits(unsigned tmkClrCWBits);

Parameters:

tmkClrCWBits – mask resetting the specified TMK user's bits to 0: for TMK400, TMKMPC, RTMK400 devices, bits CWB0 and CWB1 (correspond to bits 5 and 6 of the Control Word Register; see devices technical specification).

Returned value:

N/A

Actions to be completed:

The function resets the set user’s bits of the selected TMK. Calling bcreset and rtreset functions sets the user’s bits to zero. Using the function makes sense only for the devices where the specified bits are hardwired. These bits are not used in the standard devices.

tmkconfig [DOS]

int tmkconfig(int tmkNumber, unsigned tmkType,

unsigned tmkPorts1, unsigned tmkPorts2,

char tmkIrq1, char tmkIrq2);

Parameters:

tmkNumber – number of the device being configured, starting with 0;

tmkType – type of the installed device:

TMK400 - (compatible with TMK-400 device) a nonredundant TMK device, DPRAM 8Кx16, programmable RT address, two base port addresses (0x100, 0x110 standard), one interrupt line (IRQ10 standard);

TMKMPC - (compatible with MTMK-400 or TMK-MP devices) a nonredundant TMK device, DPRAM 2Кx16, hardwired RT address in the BUS, one base port address (0x100 standard), one interrupt line (IRQ5 standard);

RTMK400 - (compatible with RTMK-400 device) a dual redundant TMK, DPRAM 2К or 8K, programmable RT address in the BUS, one base port address (0x100 standard), one interrupt line (IRQ10 standard);

TMKX - (compatible with all ISA devices of TX1, TX6, TE1, TE6, AMKO series)

a dual redundant TMK, DPRAM 2K or 8К or 16K, programmable RT address in the BUS (except the instrumentation bit mode), one base port address (0x160 standard), one interrupt line (IRQ10 standard);

TMKXI - (compatible with all PCI devices of TE1, TE6 series) a dual redundant TMK, DPRAM 16Kx16, programmable RT address in the BUS (except the instrumentation bit mode);

tmkPorts1 – first base address of the ports for the ISA devices or the number of this type PCI card in the PC for the PCI devices;

tmkPorts2 – second base address of the ports for the ISA devices or the number of the device on the PCI card with several devices;

tmkIrq1 – number of the first interrupt line (0 - 15) of the ISA devices, not used for PCI devices;

tmkIrq2 – number of the second interrupt line (0 - 15), not used starting with the driver version 4.50.

Returned value:

0 – driver configuration has been completed,

TMK_BAD_NUMBER – invalid device number has been specified,

TMK_BAD_TYPE – invalid device type has been specified,

TMK_BAD_IRQ – invalid interrupt line number has been specified,

TMK_PCI_ERROR – incorrect attempt to configure PCI device.

Actions to be completed:

The function customizes the driver to the certain port address values and numbers of the interrupt lines for the device with the specified number. Non-used parameters (for example, numbers of the second interrupt line for the devices using one line) should take the value of 0xFFFF (for unsigned) or 0xFF (for char). Calling the function with the invalid parameter values results in erratic situations with TMK_BAD_NUMBER, TMK_BAD_TYPE, TMK_BAD_IRQ error codes. After the function completes, the device being configured remains selected for operation.

tmkconfig [Win, Linux]

int tmkconfig(int tmkNumber);

Parameters:

tmkNumber – number of the device being configured, starting with 0;

Returned value:

0 – device has been successfully configured,

TMK_BAD_NUMBER – invalid device number has been specified.

Actions to be completed:

The function connects the device with the specified number to the calling process. If the device with this number does not exist or has been already connected to another process error code will be returned. After the function completes, the device being configured remains selected for operation.

tmkdeferrors [DOS]

void tmkdeferrors(void (far* UserErrors)());

Parameters:

UserErrors – defined user’s function called when an error occurs while driver operation (when an erratic situation occurs); the function must be defined as the far-function returning void type with no parameters; error code is contained in the tmkError variable.

Returned value:

N/A

Actions to be completed:

The function defines for the driver the name of the defined user’s function which will be called each time when the error occurs while driver operation. The function is designed for debug purposes only, the defined user’s function can print the appropriate message.

tmkdefevent [Win]

void tmkdefevent(HANDLE hEvent, BOOL fEventSet);

Parameters:

hEvent – Windows event, earlier created by calling Win32 API CreateEvent function, zero value eliminates the earlier set event;

fEventSet – must be equal to TRUE.

Returned value:

N/A

Actions to be completed:

The function defines (or cancels) for the selected device the Windows event, which the driver will use to inform the process about receiving interrupts from the device. To get the appropriate information about the reason of the interrupts, the tmkgetevd function should be used.

tmkdefirq [DOS]

int tmkdefirq(char pcIrq);

Parameters:

pcIrq – number of the user’s interrupt line (0 - 15).

Returned value:

0 - the function has been successfully completed;

TMK_BAD_IRQ – invalid number of the interrupt line has been specified.

Actions to be completed:

The function allows setting the additional interrupts used in certain systems that must be masked along with masking the interrupts received from TMK devices while receiving the TMK devices interrupts and calling tmksave function.

tmkdone

void tmkdone(int tmkNumber);

Parameters:

tmkNumber – number of the TMK device with which the driver completes working (to specify all the devices ALL_TMKS constant may be used).

Returned value:

N/A

Actions to be completed:

The function must be called at the end of operation with the TMK devices. The main designation of the function in DOS is to restore the system of PC interrupts.

tmkgetcwbits

unsigned tmkgetcwbits();

Parameters:

N/A

Returned value:

State of user’s bits of the ТМК: for TMK400, TMKMPC, RTMK400 devices, bits CWB0 and CWB1 (correspond to bits 5 and 6 of the Control Word Register; see devices technical specification).

Actions to be completed:

The function returns the current state of the user’s bits of the selected ТМК, the bits having been set earlier with tmksetcwbits and tmkclrcwbits function. When bcreset and rtreset functions are called the user’s bits are set to 0. Calling the function makes sense only for the devices where the specified bits are used harwirely. These bits are not used in the standard devices.

tmkgetevd [Win, Linux]

void tmkgetevd(TTmkEventData *pEvD);

Parameters:

pEvD – reference to the existing structure TTmkEventData.

Returned value:

N/A

Actions to be completed:

The function populates the structure located at the specified address with the data of the next interrupt received from the selected interrupt device.

tmkgetmaxn

int tmkgetmaxn();

Parameters:

N/A

Returned value:

Maximum allowed number of the TMK device for this driver.

Actions to be completed:

The function returns the maximum number of the device that can be set with tmkconfig and tmkselect functions.

tmkgetmode

unsigned tmkgetmode();

Parameters:

N/A

Returned value:

Device operation mode:

BC_MODE – Bus Controller mode;

RT_MODE – Remote Terminal mode;

MT_MODE – Monitor mode;

UNDEFINED_MODE – mode not specified.

Actions to be completed:

The function returns the value describing the operation mode of the selected TMK device.

tmkiodelay [DOS]

unsigned tmkiodelay(unsigned IODelay);

Parameters:

IODelay – input/output delay. Constant GET_IO_DELAY can be defined for reading the current value only.

Returned value:

Input/output delay value for the moment of function call.

Actions to be completed:

The function defines the input/output delay for TMK400, RTMK400, TMKMPC devices (see pentium.doc file) and allows reading the current delay value.

tmkrestore [DOS]

void tmkrestore();

Parameters:

N/A

Returned value:

N/A

Actions to be completed:

The function restores the data saved by calling tmksave from the driver internal variables: number of the selected TMK device and its state: base number for the BC, subaddress number (and its locking mode) for the RT. The function also unmasks all the interrupts of the TMK devices and the interrupts set with tmkdefirq function. The function main designation is to be used in pair with tmksave function in the user’s functions for processing of the additional user’s interrupts (those interrupts being additional to TMK devices interrupts), when it is necessary to temporary save and restore the driver state and mask the TMK devices interrupts. However, this pair of functions may be used to solve the specified tasks even during normal program execution.

tmksave [DOS]

void tmksave();

Parameters:

N/A

Returned value:

N/A

Actions to be completed:

The function saves the number of the selected TMK device and its state in the internal driver variables: base number for the BC, subaddress number (and its locking mode) for the RT, and also masks all the TMK devices interrupts and the interrupts set with tmkdefirq function.

The function main designation is to be used in pair with tmkrestore function in the user’s functions for processing the additional user’s interrupts (those interrupts being additional to TMK devices interrupts), when it is necessary to temporary save and restore the driver state and mask the TMK devices interrupts. However, this pair of functions may be used to solve the specified tasks even during normal program execution. However, tmksave calls nesting is not allowed as well as calling tmksave while processing the standard driver interrupts (in UserNormBC, UserExcBC, UserCmdRT, UserErrRT functions), as all the values that have been saved earlier will be lost.

tmkselect

int tmkselect(int tmkNumber);

Parameters:

tmkNumber – number of the TMK device selected for operation.

Returned value:

0 - the function has been successfully completed;

TMK_BAD_NUMBER – invalid device number has been specified;

TMK_BAD_TYPE – the device has not been configured.

Actions to be completed:

The function leaves the device with the specified number as the selected TMK device. Specifying the invalid number results in erratic situation with TMK_BAD_NUMBER error code. If before tmkselect is called the device has not been configured with tmkconfig function, the erratic situation with TMK_BAD_TYPE error code will occur.

tmkselected

int tmkselected();

Parameters:

N/A

Returned value:

Number of the selected device.

Actions to be completed:

The function returns the number of the TMK device earlier selected with tmkconfig or tmkselect functions.

tmksetcwbits

void tmksetcwbits(unsigned tmkSetCWBits);

Parameters:

tmkSetCWBits – mask setting the specified TMK user’s bits to 1: for TMK400, TMKMPC, RTMK400 devices, bits CWB0 and CWB1 (correspond to bits 5 and 6 of the Control Word Register; see devices technical specification).

Returned value:

N/A

Actions to be completed:

The function sets the specified user’s bits of the selected TMK. Calling bcreset and rtreset functions sets the bits to zero. Calling the function makes sense only for the devices where the bits were hardwired. These bits are not used in the standard devices.

tmkundefirq [DOS]

int tmkundefirq(char pcIrq);

Parameters:

pcIrq – number of the user’s interrupt line (0 - 15).

Returned value:

0 - the function has been successfully completed;

TMK_BAD_IRQ – invalid number of the interrupt line has been specified.

Actions to be completed:

The function disables the control over the interrupt line with number pcIrq, having been set earlier with tmkdefirq function.

tmkwaitevents [Linux]

int tmkwaitevents(int mask, int wait);

Parameters:

mask – bit mask of the expected interrupts, bits 0 to 30 correspond to interrupts received from the devices with numbers 0 to 30;

wait – interrupt waiting time in milliseconds, 0 – immediate break without waiting, -1 – infinite waiting.

Returned value:

Negative value – error code,

Positive value – bit mask of the interrupts received,

Zero value – no interrupts.

Actions to be completed:

The function waits for the interrupts defined with the mask parameter during the time interval defined with the wait parameter. Only the interrupts of the devices earlier involved in tmkconfig function calls can be defined in the interrupt mask.

**************************************************************************

Macros

**************************************************************************

CW(ADDR,DIR,SUBADDR,NWORDS)  - generation of the data message COMMAND

CWM(ADDR,COMMAND) – generation of the mode COMMAND with 11111 tag

CWMC(ADDR,CI,COMMAND) – generation of the mode COMMAND with the tag set with CI

**************************************************************************

Constants

**************************************************************************

Maximum and minimum supported TMK type:

MAX_TMK_TYPE

MIN_TMK_TYPE

ТМК device types:

TMK400

TMKMPC

RTMK400

TMKX

TMKXI

Operation modes of the TMK devices:

BC_MODE			BC mode

RT_MODE			RT mode

MT_MODE			MT mode

UNDEFINED_MODE	mode not defined

Constant used to select all devices when calling tmkdone

ALL_TMKS

TMK user’s bits:

CWB0

CWB1

Message formats in the BUS (control codes for the BC):

DATA_BC_RT		BC->RT format

DATA_BC_RT_BRCST	BC->RT format in the broadcast mode

DATA_RT_BC		RT->BC format

DATA_RT_RT		RT->RT format

DATA_RT_RT_BRCST	RT->RT format in the broadcast mode

CTRL_C_A			COMMAND-STATUS format

CTRL_C_BRCST		COMMAND format in the broadcast mode

CTRL_CD_A			COMMAND+DATA-STATUS format

CTRL_CD_BRCST		COMMAND+DATA format in the broadcast mode

CTRL_C_AD			COMMAND-STATUS+DATA format

Selecting the BUS in the redundant BC:

BUS_A

BUS_B

or

BUS_1

BUS_2

Bit masks of the word of message result in the BC [DOS]:

ERAO_MASK	RT address error in STATUS

MEO_MASK		Manchester-2 code error

IB_MASK		Set bit in the STATUS

TO_MASK		STATUS timeout error (no response) or DATA timeout error (no DATA or little DATA)

EM_MASK		DPRAM access error (on TMK400, RTMK400, TMKMPC only)

EBC_MASK		self-checking error while transmitting or composite error while receiving

Bit masks of the word of message result in the BC [Win, Linux]:

S_ERAO_MASK	RT address error in STATUS

S_MEO_MASK	Manchester-2 error code

S_IB_MASK		Set bit in the STATUS

S_TO_MASK	STATUS timeout error (no response) or DATA timeout error (no DATA or little DATA)

S_EM_MASK	DPRAM access error (on TMK400, RTMK400, TMKMPC only)

S_EBC_MASK	self-checking error while transmitting or composite error while receiving

Command and status words bits masks and field masks:

NWORDS_MASK	field of the word number

CMD_MASK		mode command

SUBADDR_MASK	subaddress field

CI_MASK		setting the mode COMMAND (subaddress field contains all 1)

HBIT_MASK		instrumentation bit

RT_DIR_MASK	direction bit

ADDRESS_MASK	RT address bits

RTFL_MASK	STATUS bit "Terminal Flag"

DNBA_MASK	STATUS bit "Dynamic Bus Control"

SSFL_MASK	STATUS bit "Subsystem Flag"

BUSY_MASK	STATUS bit "Subsystem Busy"

BRCST_MASK	STATUS bit "Broadcast Command Received"

NULL_MASK	reserved STATUS bits

SREQ_MASK	STATUS bit "Service Request"

ERROR_MASK	STATUS bit "Message Error"

Masks for setting STATUS bits in the RT (rtsetanswbits, rtclranswbits, rtgetanswbits):

SREQ		sets the STATUS bit "Service Request"

BUSY		sets the STATUS bit "Subsystem Busy"

SSFL		sets the STATUS bit "Subsystem Flag"

RTFL		sets the STATUS bit "Terminal Flag"

DNBA		allows setting STATUS bit "Dynamic Bus Control"

COMMAND bit “receiving/transmitting” values:

RT_TRANSMIT	RT transmits

RT_RECEIVE	RT receives

Bit "Message Error" of the RT word of state:

RT_ERROR_MASK

Flag bit masks in the RT flag word:

RT_FLAG

RT_FLAG_MASK

Bit masks of the RT operation modes:

RT_HBIT_MODE		switches on the mode with instrumentation bit

RT_FLAG_MODE		switches on the mode with flags

RT_BRCST_MODE	switches on the mode of receiving broadcast commands

Bit masks of the TMK interrupt modes

RT_DATA_BL	switches on blocking the interrupts upon receiving/transmitting RT data

RT_GENER1_BL	switches on blocking the RT interrupts upon generation over BUS A

RT_GENER2_BL	switches on blocking the RT interrupts upon generation over BUS B

BC_GENER1_BL	switches on blocking the BC interrupts upon generation over BUS A

BC_GENER2_BL	switches on blocking the BC interrupts upon generation over BUS B

MT_GENER1_BL	switches on blocking the MT interrupts upon generation over BUS A

MT_GENER2_BL	switches on blocking the MT interrupts upon generation over BUS B

TMK_IRQ_OFF	switches off ISA interrupts of the TMKX devices

Field masks and bit masks of the extended control code

CX_CC_MASK	message format field in BUS

CX_CONT_MASK	chain continuation bit

CX_BUS_MASK	BUS defining bit

CX_SIG_MASK	Signal interrupt bit

CX_INT_MASK	bit of masking the chain stop upon error (МТ)

Bit values of the extended control code

CX_CONT		sets chain continuation

CX_STOP		sets chain stop

CX_BUS_0		sets BUS A

CX_BUS_A		sets BUS A

CX_BUS_1		sets BUS B

CX_BUS_B		sets BUS B

CX_NOSIG		sets the absence of the signal interrupt

CX_SIG		sets the signal interrupt

CX_INT		sets the chain stop upon error or set bit in the STATUS (МТ)

CX_NOINT		sets ignoring the error or set bit in the STATUS (МТ)

Field masks and bit masks of the extended word of state

SX_ERR_MASK	Extended error code field

SX_IB_MASK	set bit in the STATUS

SX_G1_MASK	generation over BUS A

SX_G2_MASK	generation over BUS B

SX_K2_MASK	error in the 2nd STATUS (МТ)

SX_K1_MASK	error in the 1st STATUS (МТ)

SX_SCC_MASK	message format code field (МТ)

SX_ME_MASK	integrated error tag in the message (МТ)

SX_BUS_MASK	BUS number (МТ)

Extended error code values

SX_NOERR		no error

SX_MEO		Manchester-2 code error

SX_TOA		STATUS timeout (no response)

SX_TOD		DATA timeout error (number of DATAS is less than specified)

SX_ELN		error, number of DATAS is higher than specified

SX_ERAO		RT address error in STATUS

SX_ESYN		Sync type error

SX_EBC		echo-check error while transmitting or composite error while receiving

BUS number in the extended word of state

SX_BUS_0		BUS A

SX_BUS_A		BUS A

SX_BUS_1		BUS B

SX_BUS_B		BUS B

Constant of receiving the current timeout value in tmkiodelay

GET_IO_DELAY

RT on/off constants in rtenable

RT_ENABLE

RT_DISABLE

RT_GET_ENABLE

Command words generation macros

CW(ADDR,DIR,SUBADDR,NWORDS) - Data transfer COMMAND

CWM(ADDR,COMMAND) - Mode COMMAND

CWMC(ADDR,CI,COMMAND) - Mode COMMAND with the tag set with CI

Mode command codes

CMD_DYNAMIC_BUS_CONTROL			Dynamic Bus Control

CMD_SYNCHRONIZE					Synchronization

CMD_TRANSMIT_STATUS_WORD			Transmit STATUS word

CMD_INITIATE_SELF_TEST				Initiate self-test in RT

CMD_TRANSMITTER_SHUTDOWN			Shutdown the transmitter

CMD_OVERRIDE_TRANSMITTER_SHUTDOWN	Override transmitter shutdown

CMD_INHIBIT_TERMINAL_FLAG_BIT			Inhibit terminal flag bit

CMD_OVERRIDE_INHIBIT_TERMINAL_FLAG_BIT	Override RT flag shutdown

CMD_RESET_REMOTE_TERMINAL			Reset RT to the initial state

CMD_TRANSMIT_VECTOR_WORD			Transmit vector word

CMD_SYNCHRONIZE_WITH_DATA_WORD		Synchronize with data word

CMD_TRANSMIT_LAST_COMMAND_WORD	Transmit last command

CMD_TRANSMIT_BUILT_IN_TEST_WORD		Transmit built-in test word

Error codes:

TMK_BAD_TYPE

TMK_BAD_IRQ

TMK_BAD_NUMBER

BC_BAD_BUS

BC_BAD_BASE

BC_BAD_LENGTH

RT_BAD_PAGE

RT_BAD_LENGTH

RT_BAD_ADDRESS

RT_BAD_FUNC

BC_BAD_FUNC

TMK_BAD_FUNC

TMK_PCI_ERROR

(c) “ELCUS Electronic Company”JSC, 1995,2006.