v.1.00.

The programmer's guide

TMKLL4 v4.54 driver library (DOS),

1553bwdm.sys v2.11 WDM driver (Windows 98, ME, 2000, XP),

tmk1553b.sys v2.11 SYS driver (Windows NT 4.0),

tmk1553b v2.11 driver module (Linux kernel 2.4).

General information

The terminal of the multiplex channel (TMK) can work in one of three modes: the Bus Controller (BC), the Remote Terminal (RT) or the Monitor (MT). Earlier developed devices support operation only in BC and RT modes. More modern devices also can work in MT mode. Required mode can be set by program. Data message occurs to the channel through two-port buffer RAM (DPRAM), access to which is possible both on the side of the PC, and on the side of the multiplex channel (BUS CHANNEL). The mode of operation of the bus CHANNEL (with the instrumentation bit / without the instrumentation bit) is usually set  by soldering or setting of a jumper on the device, on some cards - by program. The instrumentation bit using influences only operation of the RT and the MT (in the mode with the instrumentation bit used the RT does not recognize BC commands with the instrumentation bit equal to 0).

All drivers give the programmer the uniform interface (though, small distinctions for different STATUSES nevertheless exist) for operation with different types of the TMK and allows to work without direct programming devices at a register level. It enables to write practically universal programs, not dependent on type of used card (unless any unique features of selected type of cards use).

DOS

The DOS low level TMKLL4 v4.54 software driver is realized as the library of functions for C/C ++ languages. Usage of memory models from TINY up to LARGE is possible. Preferable memory model is LARGE as all calls of functions of the driver are made by rules of this model. NOTICE! The driver does not work in memory model HUGE!

Linux

Tmk1553b driver for Linux is based on tmk1553b driver module. The module should be loaded with the definition of appropriate parameters via insmod command and the device file /dev/tmk1553b should be created by mknod command according to the read.me file.

One instance of the driver controls all TMK devices set in a computer. Each separate device in every moment can be under control of a single process which has captured the device by tmkconfig call.

Windows NT 4.0

TMK1553B driver for Microsoft Windows NT 4.0 is based on tmk1553b.sys driver. The tmk1553b.sys file should be placed in the directory $(WINDIR)\system32\drivers.

One driver controls all TMK devices set in a computer, the configuration of devices is stored in the Windows NT 4.0 registry. Each separate device in every moment can be under control of a single process which has captured the device by tmkconfig call.

Windows 98, ME, 2000, XP

TMK1553B driver Microsoft Windows 98, ME, 2000, XP is based on 1553bwdm.sys driver. The 1553bwdm.sys file should be placed in the directory $(WINDIR)\system32\drivers.

For every device set in a computer driver creates a unique character name to which has access only one process which has captured the device by tmkconfig call.

All

All drivers support the following device types:

- TMK400 - a nonredundant TMK, DPRAM 8Кx16 (128 bases in BC mode, 4 pages in RT mode), simultaneous switching of pages for the bus CHANNEL and the PC in RT mode, programmable RT address in the bus CHANNEL, readable RT address;

- TMKMPC - a nonredundant TMK, DPRAM 2Кx16 (32 bases in BC mode, 1 page in RT mode), hardwired RT address in the bus CHANNEL (the address is set by jumpers on the device), card is designed in MicroPC standard;

- RTMK400 - a dual redundant TMK, DPRAM 2Кx16 on old cards (32 bases in BC mode, 1 page in RT mode), or 8Kx16 on new cards (128 bases in BC mode, 4 pages in RT mode), simultaneous switching of pages for the bus CHANNEL and the PC in RT mode, programmable RT address in the bus CHANNEL, readable RT address, programmable RT operation modes (usage of the instrumentation bit in the bus CHANNEL, a mode of operation with flags or without flags, usage of broadcast addressing in commands in the bus CHANNEL);

- TMKX - a dual redundant TMK with MT mode, DPRAM 8Кx16 on old TX1-PC cards (128 bases in BC/MT modes, 4 pages in RT mode) or 2Кx16 on old TX1-MP, TX1-104 cards and on AMKO cards (32 bases in BC/MT modes, 1 page in RT mode) or 16Kx16 (256 bases in BC/MT modes, 8 pages in RT mode) on all new TX1, TX6, TE1, TE6 series cards, separate switching of pages for the bus CHANNEL and the PC in RT mode, programmable RT address in the bus CHANNEL; all new TX/TE cards having a uniform program interface and differing only by a design and size of set RAM (size of set RAM is determined by the driver automatically in tmkconfig function call): TX1-PC, TX6-PC, TX1-MP, TX1-104, TE1-PC, TE1-104, TX1-B, the AMKO;

- TMKXI - clone of cards TMKX, but for PCI bus, DPRAM 16Kx16 (256 bases in BC/MT modes, 8 pages in RT mode): TE1-PCI, TE6-PCI, TE1-PCI2, TE6-PCI2, TE1-CPCI, TE1-CPCI-2.

The driver supports operation with several physical TMK devices simultaneously. The quantity of supported devices is set at compilation of the driver: 4 for DOS, 8 for a Linux/Windows (generally, it is possible delivery of the driver of the same version for operation with other number of devices). If on a real card there are several devices (for example, TE1-PCI2, TX1-PC-2 contain on two devices, TE1-CPCI-4 - up to four devices) each such card is understood as several TMKs. Located on a card separate physical TMKs are absolutely independent one from each other and by operation do not influence in any way against each other.

All simultaneously operating TMKs can be different types and can be in any mode - the BC, the MT or the RT. Mode change of the TMK also can occur independently and at any moment.

Besides for support of multi-address RT mode virtual TMKs are defined. Virtual TMKs are realized on the basis of the special physical TMKs supporting the multi-address RT mode. These are all cards of TX6/TE6 series. A main feature of virtual TMKs that several virtual TMKs are realized on the basis of the same physical TMK. For this reason, there are some limitations on operation with such TMKs, and there is a some interference of operation of each virtual TMK on the neighbours on a parent physical TMK. For example, program change of any mode in one of virtual TMKs can result in the same change for all virtual neighbour TMKs. Virtual TMKs can work only in RT mode. If the physical TMK is configured for operation in multi-address RT mode, it can not work in standard  BC/RT/MT modes any more. Besides physical TMKs, the Windows driver supports in addition up to 31 virtual TMKs, and Linux driver - up to 23 virtual TMKs.

The configuration of the device for operation as the standard physical BC/RT/MT or several virtual RTs is carried out manually at configuration of drivers in Windows, Linux STATUSES or start of the appropriate card loading program in DOS. This configuration can not be modified by program through the driver programming interface.

DOS

The driver is program customized on the concrete configuration of used TMKs in the PC (types of the TMK, used addresses of input/output ports and a interrupt line). Together with the driver the tmkinit.c file with the functions permitting to set the configuration of the driver from a text file of the configuration is delivered.

The driver does not allow to share interrupt with other programs, so if the TMK device on PCI bus shares the interrupt with any other card, and the driver of other card serviced its interrupts, other's interrupts will not be translated in the previous handler after tmkconfig function call.

Linux, Windows

The driver is customized on the configuration of used TMKs in the PC automatically or manually at start of the driver. The program only can determine the list of attached devices and use all or a part from them.

Including the driver in programs

DOS

For usage of the driver it is necessary to include the tmkll4.h file using the #include directive in the text of the program

#include "tmkll4.h"

When using integrated environment (IDE) it is necessary to use the project file (.prj) with tmkll4.obj driver defined in it. When using the command line compiler it is necessary to point the driver in an linker option.

Linux

For usage of the driver it is necessary to use the directive

#include "ltmk.c"

to connect in the text of the program in C/C ++ language the ltmk.c file containing definitions of all constants and API functions of the tmk1553b driver.

Windows NT 4.0

For usage of the driver it is necessary to use the directive

#include "ntmk.c"

to connect in the text of the program in C/C ++ language the ntmk.c file containing definitions of all constants and API functions of the tmk1553b.sys driver.

Windows 98/ME/2000/XP

For usage of the driver it is necessary to use the directive

#include "WDMTMKv2.cpp"

to connect in the text of the program in C/C ++ language the WDMTMKv2.cpp file containing definitions of all constants and API functions of the 1553bwdm.sys driver.

Beginning and termination of the driver using

DOS

The driver is entirely built into the user program and does not require a special function call for the beginning and the termination.

Linux, Windows

The driver is a part of a status, and the program, for a gaining access to the equipment controlled by the driver, should use special functions for the beginning and the termination of interaction with the driver.

Linux

To start operation with the driver it is necessary to call TmkOpen function

int TmkOpen();

which opens the device file and makes initial settings. At successful completion the function returns zero value. At impossibility to open the driver by open call the error code is returned.

After the end of work with the tmk1553b driver TmkClose function should be called:

void TmkClose();

Windows NT 4.0

To start operation with the driver it is necessary to call TmkOpen function:

DWORD TmkOpen();

which dynamically loads the driver in memory and makes its initial settings. At successful completion the function returns zero value. At impossibility to load the driver by CreateFile call the error code obtained by GetLastError call is returned.

After the end of work with the tmk1553b driver TmkClose function should be called:

void TmkClose();

Windows 98/ME/2000/XP

To start operation with the driver it is necessary to call TmkOpen function:

DWORD TmkOpen();

which dynamic loads the driver in memory and makes its initial settings. At successful completion the function returns zero value. At impossibility to load the driver by CreateFile call a 1 code is returned.

After the end of work with the 1553bwdm driver TmkClose function should be called:

void TmkClose();

General purpose functions

Each device controlled by the driver has number from 0 up to a maximum value supported by the driver (for the DOS driver this range is equal 0 - 3, for other drivers 0 - 7). tmkgetmaxn function returns the indicated maximum value being constant for the given driver.

In the beginning of operation it is necessary for program to enumerate devices which it is going to work with, and in DOS also to set their configuration in the PC. It can be done with tmkconfig function called separately for each device.

DOS

int far tmkconfig(int tmkNumber,

unsigned tmkType,

unsigned tmkPorts1,

unsigned tmkPorts2,

char tmkIrq1,

char tmkIrq2);

The function puts the defined type of the device, used addresses of input/output ports and used interrupt lines in correspondence to a device logical number. At execution of this function the driver is customized on operation with the indicated type of the device, and interrupts with defined numbers are intercepted and unmasked by the driver.

Linux

int tmkconfig(int tmkNumber);

If the device with required number is defined at start of the driver and the device to this moment is not bound to any process the function binds this device for the process called tmkconfig, and in the future this process gets exclusive access to the device. In this case tmkconfig function returns zero value. If the device is already bound to any process or not defined at all at start of the driver the non-zero error code is returned.

Windows

int tmkconfig(int tmkNumber);

If the device with required number is described in the Windows registry and the device to this moment is not bound to any process the function binds this device for the process called tmkconfig, and in the future this process gets exclusive access to the device. In this case tmkconfig function returns zero value. If the device is already bound to any process or not described at all in the registry the non-zero error code is returned.

All

The program releases devices with which operation is completed, through function call tmkdone:

int tmkdone(int tmkNumber);

The parameter can be either the concrete device number or ALL_TMKS constant - thus all devices defined earlier in tmkconfig are released. In DOS tmkdone results also in restoring states of the intercepted interrupts and their masking.

All functions of the driver operating with separate devices (except for tmkconfig and tmkdone), use the concept of the current selected device, so there is no device number in the list of such functions parameters. For selection of the current device tmkselect function, and for getting of the selected device number - tmkselected function should be used. tmkselected function, for example, can be used inside a single interrupt handler for several devices in DOS for definition of the interrupt source device. tmkconfig function also leaves selected that device for which it was called. Therefore, in case of application of the unique device, its number (anyone from 0 up to maximum) is set once by call tmkconfig. Several devices will need their switching with tmkselect calls. The state of non-selected devices does not change.

BC, RT or MT mode for the selected device is set by bcreset, rtreset or mtreset functions, accordingly. These functions return 0 in case of successful completion and value TMK_BAD_FUNC if the selected device does not support operation in the defined mode. tmkgetmode function allows to determine the mode of the selected device and returns one of three values: BC_MODE - BC mode, RT_MODE - RT mode, MT_MODE - MT mode, UNDEFINED_MODE - the mode of the device was not defined.

Some devices have in the registers unused bits (2 bits for devices TMK400, TMKMPC and RTMK400) which are hardware accessible on devices and can use in nonstandard modifications of devices. Operation with these bits is supported by tmksetcwbits, tmkclrcwbits and tmkgetcwbits functions, permitting, accordingly, to set, clear and determine a current state of user's bits. For choice of bits in these functions it is necessary to use predefined constants CWB0, CWB1 or their combination. Calls of bcreset and rtreset functions result in a zeroing of user's bits.

There is a possible problem of operation of devices TMK400, RTMK400 and TMKMPC on fast ISA bus when cycles on ISA bus can be generated by the system faster, than the equipment of cards can perform them. In the most of modern computers such problem does not arise. We mark problems on the very first chip sets for Intel Pentium, AMD K5, Cyrix686+ processors. In some chip sets rate of ISA bus is regulated through BIOS. If such regulation is not present to reduce a generation rate of access cycles to cards it is possible to use tmkiodelay function, allowing to set delay for access to ports of cards TMK400, RTMK400, TMKMPC. For cards such as TMKX/TMKXI delays are not required, and tmkiodelay function calls for these cards are ignored by the driver.

The function receives the unique parameter specifying number of cycles of delay in commands of access to ports of the current selected device, and returns the value of delay existing at the moment of call of this function. A range of valid values of delay is from 1 up to 65534. Call tmkiodelay(0) is equivalent to a call tmkiodelay(1). Value 65535 is reserved for constant GET_IO_DELAY which allows to find out current values of delays without their variation. For each device the delays can be defined.

DOS

Besides  the tmktest.c file contains TMK_TuneIODelay function which experimentally allows to set up delay for current selected device, performing tests of RAM of the device in BC mode at various values of delays. The function gets the unique parameter specifying a maximum value of delay. Call TMK_TuneIODelay(0) uses a maximum value of delay by default (50). The maximum value allows to limit time of selection at function call for absent (incorrectly defined in the configurations) devices.

As result the function returns 0 at successfully set up delay or constant BC_BAD_RAM if in a defined range of delays it was not possible to set up delay at which card would work without failures. After call TMK_TuneIODelay card is in BC mode.

Linux, Windows

The driver for Linux and Windows at start always automatically calls internal variant TMK_TuneIODelay for devices TMK400, RTMK400, TMKMPC, therefore programs always work with optimal adjustments of rate of the interface.

All

By call of functions of the driver practically all parameters able to affect correctness of operation of functions are checked by the driver. At that time one of two operations is performed: or value is adjusted by default by masking of all "superfluous" bits for the given function, and further the function is performed with the corrected value, or value which can not be corrected thus, completely checked on admissibility, and if it does not satisfy to criteria of correctness, there is an erratic situation, and the function is aborted. For processing such erratic situations in the driver the following rules are exist.

DOS

There is a variable tmkError accessible to the programmer, and each function in which the erratic situation can arise, writes this variable (the functions correcting default settings do not change tmkError value). If there were no errors, tmkError has zero value, otherwise - a non-zero error code is written. Besides the programmer can set via tmkdeferrors function the function of response to an error which will be called by the driver every time at the moment of detection of an error of the parameter. Such variant is expedient for using only at debugging programs as the majority of errors is caused by invalid parameters of functions because of software failures. However, there is a number of parameters which correctness depends on the current configuration. Functions with such parameters (configuration, device selection, BC/MT base definition, RT page definition etc.) write tmkError and return as result the value duplicating tmkError.

Linux

Both the driver, and its ltmk.c interface include the necessary code for support of correct values tmkError similar to the DOS driver. But, by default, all operation with tmkError in ltmk.c is disconnected. Generally, possibility of the analysis of a return code of those functions where value tmkError is duplicated in a return code is supposed only (configuration, device selection, BC/MT base definition, RT page definition etc.). If necessary it is possible to include complete tmkError support in ltmk.c, by decommenting line #define USE_TMK_ERROR in the ltmk.c file. tmkdeferrors function in Linux driver is not realized.

Windows

Interfaces of both Windows drivers do not contain the code of support of correct values tmkError. Possibility of the analysis of a return code of those functions where value tmkError is duplicated in a return code is supposed only(configuration, device selection, BC/MT base definition, RT page definition etc.). tmkdeferrors function in Windows drivers is not realized.

Interrupt processing

In all modes the device generates interrupts. Each interrupt is handled by the driver and then the interrupt information is transmitted to the user's program. The mechanism of an information communication is various for different STATUSES. In DOS the driver directly from the interrupt handler calls the user's code which has been made out as special functions. Thus, the programmer has possibility to build the code directly in the handler. In other STATUSES the interrupt handler is completely built-in in the code of the driver performed at a level of a kernel of STATUS, and has no possibility to give control to any user's code at the moment of interrupt. For decrease of influence of delays of switching on the user's code at interrupt processing in multitask STATUSES the driver includes a buffer for 512 interrupts.

Types of the interrupts processed by the driver and the data used for each type of interrupt are listed below. Names of types of interrupts (bcIntNorm etc.) in this case are only conventional and used only in this text.

bcIntNorm - interrupt on normal completion of single message in BC mode. The data: wResult == 0 (a word of message result equal to zero).

bcIntExc - interrupt on completion of single message with an exclusive situation in BC mode. The data: wResult (a word of message result), wAW1, wAW2 (status words).

bcIntX - interrupt on message(s) completion (break) in BC mode. The data: wResultX (the extended word of message result), wBase (base of break).

bcIntSig - signal interrupt in BC mode. The data: wBase (current base).

rtIntCmd - interrupt on mode command in RT mode. The data: wCmd (the mode code of mode command with a mask 0x41F).

rtIntErr - interrupt on message error in RT mode. The data: wStatus (a word of a state of the RT).

rtIntData - interrupt on data command in RT mode. The data: wStatus (a word of a state of the RT).

mtIntX - interrupt on break in MT mode. The data: wResultX (the extended word of result), wBase (base of break).

mtIntSig - signal interrupt in MT mode. The data: wBase (current base).

DOS

Before the beginning of operation the program should register in the driver the interrupt handlers for all modes and interrupts which it is planned to use. An example of the description of prototypes of all user's interrupt functions and their registrations is below.

void far BCIntNorm(unsigned wResult, unsigned dum1, unsigned dum2);

void far BCIntExc(unsigned wResult, unsigned wAW1, unsigned wAW2);

void far BCIntX(unsigned wResultX, unsigned wBase);

void far BCIntSig(unsigned wBase);

void far RTIntCmd(unsigned wCmd);

void far RTIntErr(unsigned wStatus);

void far RTIntData(unsigned wStatus);

void far MTIntX(unsigned wResultX, unsigned wBase);

void far MTIntSig(unsigned wBase);

bcdefintnorm(BCIntNorm);

bcdefintexc(BCIntExc);

bcdefintx(BCIntX);

bcdefintsig(BCIntSig);

rtdefintcmd(RTIntCmd);

rtdefinterr(RTIntErr);

rtdefintdata(RTIntData);

mtdefintx(MTIntX);

mtdefintsig(MTIntSig);

For interrupt bcIntNorm in DOS two dummy parameters that allows to use the common handler with bcIntExc are added.

Having determined a reason of interrupt, the driver calls one of the defined user's functions appropriate to a reason of interrupt. Thus the driver executes the following operations. The current selected device number is saved, the device which has produced interrupt become the current selected device, the state of interrupted device is partially saved, the driver prepares interrupt data according to a reason of interrupt, the user's function is called. In this moment all processor interrupts are disabled because any interrupt resets in 0 a flag of interrupt in the PSW. The driver does not support nested interrupts (in the sense that only single saving of a state exists, and nested interrupts from other devices would result in loss of the saved parameters to the previous interrupts). The user's function should perform only minimum necessary operation on registration of interrupt and should not enable interrupts of the processor. After return from the user's function the command of the end of interrupt is sent to an interrupt controller of the PC and the exit from interrupt is carried out.

As situations are possible when restoring of a state of devices after interrupt is not necessary, a state restoring is up to the programmer. For this purpose there are bcrestore, rtrestore and mtrestore functions which are restoring a state of the device and the selected device number saved in the beginning of BC, RT or MT interrupt. If necessary these functions should be called at the end of the user's function.

If it is not enough to have built-in resources of saving of a state at interrupts (for example if it is necessary to restore a state of several devices), the driver gives possibility of "hand" programming of required procedures. For determining of a current state of devices following functions are intended: tmkselected, tmkgetmode, bcgetbase, mtgetbase, rtgetsubaddr, - that enables in subsequent to restore modified parameters.

Windows

In Windows drivers operation with interrupts is carried out through the mechanism of Win32 events. First of all, the process should report to the driver the handle of used event for a current selected TMK through function call tmkdefevent to which has as the first parameter the identifier of Win32 event obtained from Win32 call CreateEvent:

void tmkdefevent(HANDLE hEvent, BOOL fEventSet);

Zero value hEvent cancels usage of event. The parameter fEventSet equal TRUE indicates necessity of switching of event by the driver through calls SetEvent, and value FALSE - through calls PulseEvent. Notice! In the current version of the driver the value fEventSet is ignored (default is TRUE). Besides it is recommended to call ResetEvent function directly ahead of call tmkdefevent.

Linux

In tmk1553b driver operation with interrupts is carried out as follows. There is the concept of an interrupt mask of cards which is a 32-bit number in which each of 31 low order bits corresponds to interrupt from card with number of this bit, i.e. bit 0 is for interrupt of the TMK 0, bit 1 is for interrupt of the TMK 1 etc. Thus, today the driver can work with a maximum 31 device.

For obtaining the information on the existing interrupts or for waiting for any interrupts the process can call tmkwaitevents function:

int tmkwaitevents(int mask, int wait);

Mask parameter is an interrupt mask the process interested in. If at the moment of function call there are the unprocessed interrupts satisfyiing an indicated mask the function immediately returns and as result produces a mask of existing unprocessed interrupts. If at the moment of tmkwaitevents call unprocessed interrupts are not present, the behaviour of the function depends on wait parameter which can receive values 0,-1 or positive value. At value 0 there is an immediate exit from the function (accordingly, a mask of waiting interrupts on the function return will be equal 0 if waiting interrupts are not present), and at value -1 function will wait for an interrupt satisfying a mask. If a positive value is in wait parameter the function realizes it as a maximum timeout in milliseconds. In this case the exit from the function is made either on receiving interrupts or on the expiration of a timeout. The negative tmkwaitevents return code is an error code.

Linux, Windows

After receiving interrupt the process can request the data structure, describing the interrupt becoming a reason of event. For this purpose tmkgetevd function is intended:

void tmkgetevd(TTmkEventData *pEvD);

The pointer to structure TTmkEventData is transmitted as the parameter and the driver fills the structure with appropriate values:

typedef struct

{

int nInt;

unsigned short wMode;

union

{

struct

{

unsigned short wResult;

unsigned short wAW1;

unsigned short wAW2;

} bc;

struct

{

unsigned short wBase;

unsigned short wResultX;

} bcx;

struct

{

unsigned short wStatus;

unsigned short wCmd;

} rt;

struct

{

unsigned short wBase;

unsigned short wResultX;

} mt;

struct

{

unsigned short wStatus;

} mrt;

};

} TTmkEventData;

Memory for the structure should be allocated beforehand in the process, the driver only fills it out! First two fields define the type of interrupt and additional data fields appropriate to this type. The datas are the same, as in DOS.

| bcIntNorm | wMode=BC_MODE, nInt=1, data bc.wResult |
| --- | --- |
| bcIntExc | wMode=BC_MODE, nInt=2, data bc.wResult, bc.wAW1, bc.wAW2 |
| bcIntX | wMode=BC_MODE, nInt=3, data bcx.wResultX, bcx.wBase |
| bcIntSig | wMode=BC_MODE, nInt=4, data bcx.wBase |
| rtIntCmd | wMode=RT_MODE, nInt=1, data rt.wCmd |
| rtIntErr | wMode=RT_MODE, nInt=2, data rt.wStatus |
| rtIntData | wMode=RT_MODE, nInt=3, data rt.wStatus |
| mtIntX | wMode=MT_MODE, nInt=3, data mt.wResultX, mt.wBase |
| mtIntSig | wMode=MT_MODE, nInt=4, data mt.wBase |

If the situation is possible, when before completion of interrupt processing by the process new interrupts can arise (and it is usual indeed) it is necessary to call tmkgetevd in cycle until in obtained structure TTmkEventData the interrupt number field does appear zero (values of other fields of structure TTmkEventData in a case nInt=0 are not defined). After that again it is possible to wait for interrupt.

Also, it is necessary to take into account, that new interrupt can arise in time of cycle of tmkgetevd calls for processing the previous interrupts; then at processing new interrupt the first tmkgetevd call will give zero interrupt number as this interrupt data were read earlier.

The Bus Controller

In BC mode DPRAM is divided into bases - sections in length of 64 words. For the message definition it is necessary to prepare in any base the message according to the selected format (fig. 1) to set the control code for the selected format and to start message from this base. After start of message according to the command word (COMMAND) (command words in RT-RT mode) and the control code the BC reads from DPRAM and transmits necessary COMMANDS and data words (DATA) in the bus CHANNEL and receives from the channel all transmitted by the RT (both RTs in RT-RT mode) status words (STATUS) and DATA. Two variants of completion of message are possible: normal completion and completion with an exclusive situation. Normal completion means, that at message the BC has not detected errors and in status words there are no set bits. Completion with an exclusive situation means, that either even in one of received STATUSES there are set bits (that demands program response), or during message in the channel the BC has detected an error. Depending on type, the TMK hardware can determine a different set of errors.

All TMKs in BC mode can form the following tags:

- IB - there are set bit(s) in received STATUS(ES);

- EBC - an echo check error (can arise at a fault of the BC, at a fault of the RT - bus jam, at bus wiring failure, by operation of several BCs in the bus CHANNEL);

- MEO - an error of the Manchester code at receiving (arises at mismatch of received words from the bus CHANNEL to the standard - an incorrect sync signal, a parity error);

- TO - an error of timing in the bus CHANNEL (arises when time intervals between words in the message do not correspond to the standard - an error of a timeout to the answer of the RT and an error of continuity of the answer of the RT, and also at mismatch of number of DATA transmitted the BC expected - an error of number of words, for example if the RT is busy); more modern devices (TMKX/TMKXI) also allow to distinguish errors of a timeout of STATUS (TOA) and errors of a timeout of DATA (TOD);

- ERAO - an error of the RT address (arises when an RT address of STATUS does not coincide with the RT address defined in the COMMAND).

the TMK of TMK400, RTMK400, TMKMPC types can form tag:

- EM - an error of DPRAM access (arises if at the moment of receiving or transmission there was an inadmissible delay of access to DPRAM and a word was lost);

the TMK of TMKX, TMKXI types can form tags:

- ELN - an error of exceeding of number of words (arises if the RT answers with number of words more, than it is defined in the command);

- EBC - a composite error at the receiving, combined with an echo check error (can arise if the BC at receiving has simultaneously detected some errors).

DOS

the TMK of TMKX, TMKXI types can form tags also:

- G1 - generation in BUS A (the additional asynchronous bit - arises in case of detection of continuous signal in BUS A (bus A jam) longer than 768 us and it either is reset at end of generation, or can be reset by program reset of the device);

- G2 - generation in BUS B (the additional asynchronous bit - arises in case of detection of continuous signal in  BUS B (bus B jam) longer than 768 us and it either is reset  at end of generation, or can be reset by program reset of the device);

There is no support for processing of these tags in any driver (DOS, Linux, Windows). As such processing should be performed extremely in the interrupt handler there is a possibility of implementation of user's processing only in the DOS driver. Therefore in Linux and in Windows unconditionally, and also in DOS if such user's processing is not realized, the programmer should not enable the TMK hardware to produce these tags.

All

During message processing in the bus CHANNEL it is possible to work with other bases of the BC - to prepare new messages and process previous messages.

The TMK device can be switched in BC mode with simultaneous reset of the device or this mode can be reinitialized via bcreset function call.

Some devices (TMKX) allow customization of interrupt modes of the BC, in particular, the disabling of interrupts on bus jam, and also disconnecting of the interrupt request line of the device. For handle of such devices the following constants can be used:

- BC_GENER1_BL - block interrupts on generation in BUS A;

- BC_GENER2_BL - block interrupts on generation in BUS B;

- TMK_IRQ_OFF - disconnect the interrupt request line of the device.

For customization of such devices in BC mode bcdefirqmode and bcgetirqmode functions are intended. bcdefirqmode function sets the mode, defined by a combination of bits BC_GENER1_BL, BC_GENER2_BL and TMK_IRQ_OFF, for selected devices and writes it in an internal variable of the driver, and bcgetirqmode function reads this variable. The default (BC_GENER1_BL|BC_GENER2_BL) value specifying compatibility mode of all devices is always written in this variable at bcreset function call.

For operation with DPRAM the programmer at first should select base with bcdefbase function. bcgetmaxbase function returns maximum accessible base number on the selected device (numbers of bases lay in a range from 0 up to this maximum number). To determine number of selected base it is possible to use bcgetbase function, which main usage - user's interrupt functions. After base selection all operation with DPRAM occurs in this base before selection of other base with bcdefbase.

DOS

At interrupts from the BC or at tmksave function call in BC mode current base number is saved by the driver so the programmer can work in other bases, and at exit from interrupts with bcrestore call or by tmkrestore call base with saved number is restored as current.

All

In selected base bcputw, bcgetw functions can be used for writing, reading of words in DPRAM and bcputblk, bcgetblk functions can be used for writing, reading of blocks of words in DPRAM. With these functions in selected base the message for the bus CHANNEL is prepared and results after completion of message are read.

Command words should be formed by the programmer and to be written with the same functions. Three macros CW, CWM, CWMC are defined for command words generation from the standard fields.

Macro CW(ADDR, DIR, SUBADDR, NWORDS) is intended for creation of data transfer command words. It is possible to use the predefined constants of data direction RT_RECEIVE and RT_TRANSMIT.

Macro CWM(ADDR, COMMAND) is intended for creation of mode command words. A 11111 code is used as mode indicator.

Macro CWMC(ADDR, 0, COMMAND) allows to form mode command words with the zero code as mode indicator.

For the definition of the mode code it is possible to use constants:

CMD_DYNAMIC_BUS_CONTROL

CMD_SYNCHRONIZE

CMD_TRANSMIT_STATUS_WORD

CMD_INITIATE_SELF_TEST

CMD_TRANSMITTER_SHUTDOWN

CMD_OVERRIDE_TRANSMITTER_SHUTDOWN

CMD_INHIBIT_TERMINAL_FLAG_BIT

CMD_OVERRIDE_INHIBIT_TERMINAL_FLAG_BIT

CMD_RETURN_REMOTE_TERMINAL

CMD_TRANSMIT_VECTOR_WORD

CMD_SYNCHRONIZE_WITH_DATA_WORD

CMD_TRANSMIT_LAST_COMMAND_WORD

CMD_TRANSMIT_BUILT_IN_TEST_WORD

COMMAND with the broadcast address should contain in an address field a constant 31 (dec) = 11111 (bin).

At creation of data transfer commands  it is necessary to take into account the mode in which the bus CHANNEL will work: with the instrumentation bit or without the instrumentation bit. In the mode without the instrumentation bit it is possible to access in the RT 30 subaddresses (from 1(dec) = 00001(bin.) up to 30(dec) = 11110(bin)), and for mode commands a subaddress field should be equal 00000(bin). In the mode with the instrumentation bit the most significant bit of a field of a subaddress of the COMMAND becomes the instrumentation bit which should be always set in 1, the number of accessible subaddresses is reduced to 15 (from 0(dec) = 0000(bin) up to 14(dec) = 1110(bin), thus in a field of a subaddress values from 10000(bin) up to 11110 (bin) are formed), and for mode commands value of a field of subaddress should be equal 11111(bin).

At creation of the COMMAND without usage of macros CW, CWM, CWMC it is necessary to remember the following.

In the mode with the instrumentation bit it is possible to use constants HBIT_MASK for setting in 1 instrumentation bit and CI_MASK - for setting of mode 11111.

Value of a field of a block length of the data in the COMMAND equal 00000 (bin) sets a block length in 32 words, and other values 00001-11111 (bin)  directly correspond to a block length of 1-31 words of the data.

At creation of mode commands it is necessary to follow table 2.

At creation of mode commands it is necessary to pay attention to possibility of application of broadcast addressing. Thus each command should be applied according to the its assignment. On many standard commands the TMK device in RT mode provide the defined hardware response. Mode commands "Dynamic Bus Control", "Synchronize", "Synchronize with Data Word", " Transmit Vector Word " are used at normal system operation, and the others are intended, first of all, for error handling in the bus CHANNEL.

For BC mode two various functions and constants sets exist for sending messages in the bus CHANNEL and processing of results. The first set based on bcdefbus/bcstart functions and the standard control codes and codes of results of message works on all types of the TMK and provides single start of one message from the defined base. The second set works only on the devices supporting execution of a chain of messages (TMKX/TMKXI), and allows to set single start of one message from the defined base or a chain of messages, since the defined base. This set is based on bcdeflink/bcstartx/bcstop functions and the extended control codes and codes of results of message.

At first there is a description of operation with the first set.

In devices with dual redundant bus interface it is possible to choose one of two BUSES on which the BC will work. For this purpose bcdefbus function is intended. The bus selected by this function will be used for all messages before the following call bcdefbus. Call of this function for nonredundant bus devices results in originating an error. Constants BUS_1 and BUS_A for bus A, BUS_2 and BUS_B for bus B are defined. With bcgetbus function it is possible to find out, what bus is selected in the present moment. bcreset function selects BUS A.

The message generated in any base can be started in bus channel with bcstart function. As parameters of the function the base number and the control code for the device are specified. After start of message function immediately returns in the program, not waiting completions of message. bcstart function does not change current selected base for operation with DPRAM. Valid control codes are predefined as constants:

- DATA_BC_RT 		 - data transfer BC-RT;

- DATA_BC_RT_BRCST 	 - data transfer BC-RT in the broadcast mode;

- DATA_RT_BC 		 - data transfer RT-BC;

- DATA_RT_RT 		 - data transfer RT-RT;

- DATA_RT_RT_BRCST 	 - data transfer RT-RT in the broadcast mode;

- CTRL_C_A 		 - mode command of the format COMMAND-STATUS;

- CTRL_C_BRCST 	 - mode command of the format COMMAND in the broadcast mode;

- CTRL_CD_A 		 - mode command of the format COMMAND+DATA-STATUS;

- CTRL_CD_BRCST 	 - mode command of the format COMMAND+DATA in the broadcast mode;

- CTRL_C_AD 		 - mode command of the format COMMAND-STATUS+DATA.

After the ending of execution of the message started by bcstart, the BC produces one of two interrupts of completion of the message: bcIntNorm or bcIntExc. For both interrupts the driver forms a word of message result, and at completion with an exclusive situation also the status words received by the BC. In messages with one status word, the second status word has undefined value. At normal completion a word of result is always equal to zero. At completion with an exclusive situation the set bits in a word of result characterize a reason of such completion.

DOS

In order to have a possibility to use single user's function of completion in both cases, functions have identical prototypes, however at normal completion the function have two undefined dummy parameters instead of status words. Before call of user's functions of completion of message the driver selects that base from which message was started.

All

Various variants of completion with an exclusive situation are possible: bit IB of a word of message result is set only, and error bits are reset; bit IB of a word of message result is set and at least one error bit is set; at least one error bit in a word of message result is set even, and bit IB is reset. The constants necessary for the analysis of a word of message result:

DOS

- ERAO_MASK - a mask of bit of an error of the address of STATUS;

- MEO_MASK - a mask of bit of an error of the Manchester code at receiving;

- IB_MASK - a mask of the bit indicating presence of set bits in a STATUS;

- TO_MASK - a mask of bit of an error of a timeout at receiving;

- EM_MASK - a mask of bit of an error of access to DPRAM on the side of the channel (only for TMK400, RTMK400, TMKMPC);

- EBC_MASK - a mask of bit of an error of echo check during transmission;

- ELN_MASK - a mask of bit of an error of number of data words (number of an DATA more than defined) (only for TMKX/TMKXI);

- G1_MASK - a mask of the bit indicating detection by the receiver of the device of bus jam in BUS A (only for TMKX/TMKXI);

- G2_MASK - a mask of the bit indicating detection by the receiver of the device of bus jam in BUS B (only for TMKX/TMKXI);

Linux, Windows

- S_ERAO_MASK - a mask of bit of an error of the address of STATUS;

- S_MEO_MASK - a mask of bit of an error of the Manchester code at receiving;

- S_IB_MASK - a mask of the bit indicating presence of set bits in a STATUS;

- S_TO_MASK - a mask of bit of an error of a timeout at receiving;

- S_EM_MASK - a mask of bit of an error of access to DPRAM on the side of the channel (only for TMK400, RTMK400, TMKMPC);

- S_EBC_MASK - a mask of bit of an error of echo check during transmission;

- S_ELN_MASK - a mask of bit of an error of number of data words (number of an DATA more than defined) (only for TMKX/TMKXI);

All

The most simple variant - when bits of errors are set only. In this case message is considered invalid, status words have undefined value.

If bit IB is set only - for definition of response of the program it is necessary to analyze a status word (two status words in RT-RT mode).

The constants necessary for the analysis of a status word:

- ADDRESS_MASK - a mask of an address part of STATUS, the address of answering RT;

- ERROR_MASK - a mask of bit "Message Error", the bit is set if at least one of DATA in the message received by the RT is invalid or message is invalid or the RT has received an illegal mode command;

- HBIT_MASK - a mask of the “Instrumentation” bit, in a status word the bit should be zero always;

- SREQ_MASK - a mask of bit "Service request", bit is set for the message to an active BC about necessity of transmission or receiving of the information to the given RT; if setting of this bit can be caused by various reasons, for definition of a contents of request the BC asks from the RT a vector word (the command "Transmit Vector Word");

- NULL_MASK - a mask of bits which should be zero always;

- BRCST_MASK - a mask of bit "Broadcast Command Received", the bit is set if the valid COMMAND of the previous message was with the broadcast address; as formats of messages with the broadcast address provide no STATUS, this bit is for definition of that broadcast command was received correctly;

- BUSY_MASK - a mask of bit "Subsystem Busy", the bit is set for the indication to the BC that the RT can not transmit the data in a subsystem or obtain the data from a subsystem according to the command of the BC;

- SSFL_MASK - a mask of bit "Subsystem Flag", the bit is set if in a subsystem of the RT there is a fault, and the data requested for a subsystem, can be invalid; in case the RT can be interfaced to several subsystems, signals of tags of a fault of subsystems are combined in this bit, the BC determines a fault through usual messaging as there is no mode command, allowing to receive a word of built-in test of a subsystem;

- DNBA_MASK - a mask of bit "Dynamic Bus Control Accepted", the bit is set if the RT has accepted command of an active BC to receive control of the channel on the command "Dynamic Bus Control";

- RTFL_MASK - a mask of bit "Terminal Flag", the bit is set if the fault in the RT is detected; the BC has possibility to inhibit or override inhibit this bit by means of usage of special mode commands; for obtaining more detailed information on a fault of the RT the BC can use the command "Transmit Buil-In Test Word ".

Thus, setting of different bits in a STATUS should to cause various response of the program. For example, at set bit BUSY it is possible  to repeat the message immediately or a  later, as in these cases the message is invalid (thus probably also presence of tag of error TO (TOD) if the RT has not transmitted all required words of the data); at set bits SSFL and RTFL invalidity of the message because of failure on the side of the RT is possible; at set bit SREQ the message is valid, but the BC should react on request of service of a subsystem of the RT; at set bits BRCST and DNBA the message is valid in the event that there were reasons of their setting, either in the previous message broadcast addressing was used, or the BC tried to transmit control of the channel to the RT answering with bit DNBA and on the contrary if in the indicated cases bits BRCST or DNBA are not set, it points any erratic situation.

Set simultaneously bit IB and bits of errors in a word of message result mean, that at first the BC has received valid STATUS with set bits, and then there was any erratic situation and message was stopped. Thus the programmer can analyze STATUS (in RT-RT mode in this case if in first STATUS there are set bits second STATUS is undefined), but the message is considered invalid. The most often situation in normal system - simultaneous setting of bits IB (because of set bit BUSY of a status word) and TO, that is a subsystem of the RT is busy, the RT produces STATUS, but does not produce the requested data.

Usage only already described functions allows to form and start separate messages equally on all devices. Operation with functions which allow to start chains of messages below is described. It is the second set of functions of start of messages.

Main differences by operation with chains of messages are determined by algorithm of operation of the BC in the mode of a chain of messages. Such mode is set by setting of bit of automatic continuation in the extended control code at start of the message.

If the bit of automatic continuation is not set, the BC after finishing of the message causes interrupt and forms a word of message result. Such mode is used in bcstart function which forcibly resets bit of automatic continuation for the started message.

For start of a chain of messages bcstartx function is entered, allowing to set required value of bit of continuation in the extended control code. For each base in a chain with bcdeflink function number of the following base of a chain and the extended control code of the following base of a chain should be defined preliminary. Thus, if the bit of continuation in the control code is set, the BC, in case of normal completion of the message, _without_ generation of interrupt, reads earlier defined control code and base number for the following message in a chain and launches this message. If message on the bus CHANNEL was completed with an exclusive situation, or it was the last message of a chain, the BC stops, generates interrupt and forms a word of message result.

the BC after bcstartx calls _does not_ generate interrupts bcIntNorm/bcIntExc. Instead of these an interrupt bcIntX is formed. Through parameters of this interrupt the driver reports base number of the last message, and a word of message result, which has the _different_ format (with extended diagnostics of errors) from the format of a word of result at interrupts bcIntNorm/bcIntExc.

DOS

By call of the user's function the base of the last message (and which number is transmitted as the parameter) becomes current selected base.

All

Status words are absent in the information accompanied these interrupts, but there is the additional bcgetansw function receiving as the parameter the format of the message code and returning a double word, which low 16 bits contain the first status word in current selected base (or value FFFF if the defined format does not provide status words), and high 16 bits - the second status word (for the format of the RT - RT, or value FFFF, for other formats). bcgetansw function itself can not determine, whether status words were obtained actually; it reads DPRAM cells in which status words should be written according to the current command word of base and a value of the code of the format of the message, therefore solution on reliability of value returned by this function should be taken according to the code of message result.

The chain of messages can be linear (the last message has the extended control code with the reset bit of continuation), and can be cyclic (in this case the last message, as such, is not present - all messages have the set bit of continuation and refer against each other on a circle; a minimum example of such cycle can be the unique message having the reference to itself). At execution of a chain of messages of the BC does not produce any interrupts (interrupt is formed only at break of a chain). If it is necessary to synchronize execution of the program with transmission of concrete messages of a chain, it is possible to set in the extended control code for the necessary bases special bit of signal interrupt. If in the moment of _loading_ in the BC of the control code (i.e. directly ahead of start from base with such control code) in this code there is set bit of signal interrupt, the BC causes additional signal interrupt bcIntSig. As the parameter of this interrupt the driver transmits base number from which a message is being transmitted at the moment of call.

DOS

The current selected base in this case does not vary, though it is saved in an internal variable of the driver and can be standardly restored by call bcrestore at an exit from interrupt. As against other interrupts at an exit from signal interrupt number of the current selected device is automatically restored, therefore it is necessary to call bcrestore only if in the user's function the current base was changed, and it is necessary to restore its number, being in moment of interrupt.

All

It is necessary to mark, that in systems with dual redundant bus function call bcdefbus sets the BUS only for the messages launched by bcstart function. In the messages launched by bcstartx function, the BUS is selected separately for each base in appropriate bit from the extended control code.

Start of a chain of messages is carried out by bcstartx function having the format of call similar to bcstart format, but accepting the extended control codes. The extended control code consists from:

- Purely the control code CX_CC_MASK defining the format of the message (constants of the control code of the format completely coincide with control codes for bcstart function);

- Bit CX_BUS_MASK of the definition of the BUS (values CX_BUS_0, CX_BUS_A (or 0) select BUS A, and CX_BUS_1, CX_BUS_B - BUS B);

- Bit CX_CONT_MASK of automatic continuation (value CX_STOP (or 0) sets break of a chain after finishing of this message, and value CX_CONT sets automatic start of the following message in a chain after finishing of this message);

- Bit CX_SIG_MASK of permission of signal interrupt (value CX_NOSIG (or 0) prohibits  signal interrupt, and value CX_SIG enables signal interrupt at the moment of _loading_ (!) of the control code in the BC).

For chaining messages there is bcdeflink function having the format of call similar to bcstartx format, and specifying for current selected base the following base in a chain and the extended control code for the message in the following base of a chain. Values set by bcdeflink can be received, having called bcgetlink function returning a double word, which low 16 bits contain number of the following base in a chain, and high 16 bits - the extended control code for the following base in a chain which can be analyzed with constants of selection of fields CX_CC_MASK, CX_BUS_MASK, CX_CONT_MASK, CX_SIG_MASK. bcdeflink and bcgetlink functions work only in current selected base, that is for the definition of all chain it is necessary for each base in a chain (from the first base and until the next to last base) to execute a pair of calls bcdefbase - bcdeflink. For the last base in a linear chain bcdeflink call is not required.

If necessary it is possible to interrupt a chain of messages, having called bcstop function. This function has no parameters, and its call results in a zeroing of bit of automatic continuation in the message which in this moment is transmitted in the channel. As a result the BC after the finishing of this message stops and as usually, forms interrupt bcIntX.

The word of the extended result of the message, formed by the BC after calls bcstartx consists of several fields which can be selected and analyzed with constants:

- SX_ERR_MASK - a mask of a field of the error code containing an error code from 0 up to 7 (0 means no errors);

- SX_IB_MASK - a mask of the bit indicating presence of set bits in a STATUS;

- SX_G1_MASK - a mask of the bit indicating detection by the receiver of the device of bus jam in BUS A;

- SX_G2_MASK - a mask of the bit indicating detection by the receiver of the device of bus jam in BUS B.

It is considered, that the code of result with zero values of fields SX_ERROR and SX_IB corresponds to normal completion of the message. If field SX_ERROR contains a non-zero error code and/or bit SX_IB is set the message is considered completed with an exclusive situation. Constants of a field of an error code:

- SX_NOERR - 0 - message is completed without errors;

- SX_MEO - 1 - a parity error or an error of the Manchester code;

- SX_TOA - 2 - an incorrect timeout before status word (there is no status word);

- SX_TOD - 3 - continuity of the message (there is no word (words) of the data) is broken;

- SX_ELN - 4 - number of data words more than defined;

- SX_ERAO - 5 - the incorrect RT address in STATUS;

- SX_ESYN - 6 - an error of a sync signal type;

- SX_EBC - 7 - an error of echocheck at transmission or a combination of several errors at receiving.

Between error codes in a word of message result of bcstartx and bits of errors in a word of message result of bcstart such correspondence is set: errors SX _MEO, SX_ESYN result in setting of bit MEO_MASK (S_MEO_MASK); errors SX_TOA, SX_TOD - to setting of bit TO_MASK (S_TO_MASK); error SX_ELN - to setting of bit ELN_MASK (S_ELN_MASK); error SX_ERAO - to setting of bit ERAO_MASK (S_ERAO_MASK); error SX_EBC - to setting of bit EBC_MASK (S_EBC_MASK).

Except for definition of signal interrupts there is one more way of tracing behind base from which the BC works at present. bcgetstate function returns a double word, which low 16 bits contain base number to which there is an operation, and high 16 bits - a current word of a state. Call of this function results in reset of the not serviced signal interrupt if it in this moment was, therefore it is necessary to combine with care bcgetstate function and possibility of the definition of signal interrupts.

The Monitor

The mode of the Monitor is similar by principles of control and formats of functions to the mode of the Bus Controller. Switching of the TMK device in MT mode with simultaneous reset of the device, and reinitialization of the mode, is carried out by mtreset function. It is necessary to check value returned from the function as the device can not support the mode of the Monitor.

The devices supporting the mode of the Monitor (TMKX/TMKXI), admit customization of the interrupts mode of the MT, in particular, the disabling of interrupts on bus jam, and also disconnecting of the interrupt request line of the device. For handle of such devices constants can be used:

- MT_GENER1_BL - disable interrupts on generation in BUS A;

- MT_GENER2_BL - disable interrupts on generation in BUS B;

- TMK_IRQ_OFF - disconnect the interrupt request line of the device.

For customization of such devices in MT mode mtdefirqmode and mtgetirqmode functions are intended. mtdefirqmode function sets the mode, defined by a combination of bits MT_GENER1_BL, MT_GENER2_BL and TMK_IRQ_OFF, for the selected device and writes it in an internal variable of the driver, and mtgetirqmode function reads this variable. At function call mtreset the default (MT_GENER1_BL|MT_GENER2_BL) is always written in this variable.

In MT mode DPRAM is divided into bases - sections in length of 64 words. For operation with DPRAM the programmer at first should select base with mtdefbase function. mtgetmaxbase function returns maximum accessible base number on the selected device (numbers of bases lay in a range from 0 up to this maximum number). To find out number of selected base it is possible to use mtgetbase function, which main usage is user's interrupt functions. After selecting of base all operation with DPRAM occurs in this base before selecting of other base with mtdefbase.

DOS

At interrupts from the MT or at function call tmksave for the MT current base number is saved by the driver so the programmer can work in other bases, and at an exit from interrupts of the MT with call mtrestore or by call tmkrestore current base is restored with saved number.

All

In selected base mtputw, mtgetw functions for writing, reading of words in DPRAM and mtputblk, mtgetblk function for writing, reading of blocks of words in DPRAM work. By usual operation functions of reading from DPRAM mtgetw and mtgetblk are used only with which help the information on the messages traced by the Monitor in the bus CHANNEL can be read. Functions of writing can be used, for example, for testing DPRAM.

By operation the Monitor forms two types of interrupts: mtIntX and mtIntSig. Interruption mtIntX is caused at break of the Monitor. Through parameters of this interrupt the driver transmits base number in which the last message was written, and a word of a state of the message.

DOS

The same base becomes active. For restoring current base number before an exit from the user's function it is necessary to call mtrestore function.

All

At execution of a chain the MT does not produce interrupts (interrupt is formed only at break of the Monitor). If it is necessary to synchronize execution of the program with operation of the Monitor, it is possible to set in the control code for the necessary bases special bit of signal interrupt. If at the moment of loading in the MT of the control code (i.e. directly ahead of start from base with such control code) in this code there is a set bit of signal interrupt, the MT generates additional signal interrupt mtIntSig. As the parameter of this interrupt the driver transmits number of current start base.

DOS

The current selected base by call of the user's function does not vary, though is saved in an internal variable of the driver and can be standardly restored by call mtrestore at an exit from interrupt. As against other interrupts at an exit from signal interrupt current selected device number is automatically restored, therefore it is necessary to call mtrestore only if in the user's function the current base was changed, and it is necessary to restore its number, being in moment of interrupt.

All

For start of the Monitor it is necessary to prepare in bases with function calls mtdeflink the program of the Monitor describing a sequence of used bases, it is similar to the BC in the mode of chains of messages and then to start the Monitor with mtstartx function, having specified it as parameters number of the first base in a chain and the extended control code for it. mtdeflink function sets for current selected base the following base in a chain and the extended control code for the following base of a chain. In MT mode the extended control code includes:

- Bit CX_CONT_MASK of automatic continuation (value CX_STOP (or 0) sets break of a chain after finishing of this base, and value CX_CONT sets automatic jump to the following base in a chain after finishing of this base);

- Bit CX_INT_MASK of the disabling of interrupt of the Monitor at detection of an exclusive situation in this base - the set bits in STATUS or errors in the message in the bus CHANNEL (value CX_INT enables break of the Monitor on an exclusive situation, and CX_NOINT - forces the Monitor to proceed to the following base in a chain even if in the message the exclusive situation was detected, of course, if the bit of automatic continuation is set only);

- Bit CX_SIG_MASK of enabling of output of signal interrupt (value CX_NOSIG (or 0) disables generation of signal interrupt, and value CX_SIG enables generation of signal interrupt at the moment of _loading_ (!) of the control code in the MT).

Values set by mtdeflink can be received, having called mtgetlink function returning a double word, which low 16 bits contain number of the following base in a chain, and high 16 bits - the extended control code for the following base in a chain which can be analyzed with constants of selection of fields CX_CONT_MASK, CX_INT_MASK, CX_SIG_MASK. mtdeflink and mtgetlink functions work only in current selected base, that is for the definition of all chain it is necessary for each base in a chain (from the first base until the next to last base) to execute a pair of calls mtdefbase - mtdeflink. For the last base in a linear chain mtdeflink call is not required.

After start by mtstartx function, the Monitor enters a listening of the bus CHANNEL mode. In this mode the Monitor tries to recognize all messages which are transmitted in the channel. Control is immediately returned in the program. mtstartx function does not change current selected base for operation with DPRAM. Having recognized the message, the Monitor writes all words (command, satatus and data) by way of their arrival in that base which was indicated at start of the Monitor. After receiving of the message, the Monitor forms a word of a state on this message. Normal completion means, that at message the MT has not detected errors, and there are no set bits in status words. Completion with an exclusive situation means, that either at least in one of received STATUSES there are set bits (that demands program response), or during message in the channel the MT has detected an error. Further the Monitor can either continue operation or stop. The Monitor continues operation in the event that in the current control code the bit of automatic continuation CX_CONT is set and, either in the message it was not revealed anything resulting in an exclusive situation (is not present neither errors, nor the set bits in STATUS), or has arisen the exclusive situation, but there is set bit of the disabling of interrupt of the Monitor CX_NOINT in the current control code). If the Monitor continues operation it writes a word of a state of the message in current start base, reads number of the following start base in a chain and the extended control code for it, in this moment if in this control code the bit of signal interrupt is set, the Monitor generates mtIntSig interrupt, and then again switches in a listening of the bus CHANNEL mode with new current start base. As the parameter of mtIntSig interrupt number of current start base is transmitted. The Monitor stops if in the current control code the bit of automatic continuation CX_STOP is reset, or there is an exclusive situation and the bit of the disabling of interrupt of the Monitor CX_INT is reset. At break the Monitor does not write a word of a state of the message in base. In this case the drviver generates mtIntX interrupt. As parameters number of the last start base and a word of a state of the message written in this base are transmitted. Further the program can read with calls mtgetw, mtgetblk all words (the COMMAND, STATUS and an DATA) of messages according to the code of the format of the message, bits of a state and an error code contained in a word of a state.

DOS

By call of the user's function for mtIntX interrupt the base in which the last message was written, becomes current selected base.

All

To read a word of a state of the message for current selected base in the event that on this message the Monitor has not stopped and has written a word of a state in base, mtgetsw function is intended.

The format of a word of a state is given on fig. 5. It consists of several fields which can be selected and analyzed with constants:

- SX_BUS_MASK - a mask of the bit indicating on what bus in dual redundant bus system (SX_BUS_0, SX_BUS_A - BUS A, SX_BUS_1, SX_BUS_B - BUS B) the message is received;

- SX_ME_MASK - a mask of bit of the integrated tag of an error, this bit can be set even in the event that the field of an error code does not contain any error code, i.e. the error does not fall under one of the characteristics stipulated for this field, but nevertheless the Monitor has recognized the message erratic;

- SX_SCC_MASK - a mask of a field of the code of the format of the message (look below);

- SX_K1_MASK - a mask of the bit indicating, that the MT has found an error in the first COMMAND;

- SX_K2_MASK - a mask of the bit indicating, that the MT has found an error in the second COMMAND;

- SX_G1_MASK - a mask of the bit indicating detection by the receiver of the device of bus jam in BUS A;

- SX_G2_MASK - a mask of the bit indicating detection by the receiver of the device of bus jam in BUS B;

- SX_IB_MASK - a mask of the bit indicating presence of set bits in a STATUS;

- SX_ERR_MASK - a mask of a field of the error code containing an error code from 0 up to 7 (0 means no errors).

The message is considered normal only in the event that bit of the integrated tag of an error the message (SX_ME_MASK) and a field of an error code (SX_ERR_MASK) are equal to zero. It is important that situations are possible when the error code is equal to zero (SX_NOERR) but the bit of the integrated tag of an error nevertheless is set. If fields ERR or ME contain a non-zero error code and/or bit IB is set a message is considered completed with an exclusive situation. The constants necessary for the analysis of a word of message result:

- SX_NOERR - 0 - message is completed without errors;

- SX_MEO - 1 - a parity error or an error of the Manchester code;

- SX_TOA - 2 - an incorrect timeout before status word (there is no status word);

- SX_TOD - 3 - continuity of the message (there is no word (words) of the data) is broken;

- SX_ELN - 4 - number of data words more than defined;

- SX_ERAO - 5 - the incorrect RT address in STATUS;

- SX_ESYN - 6 - an error of a sync signal type;

- SX_EBC - 7 - a combination of several errors.

The code of the format of the message can be selected with constant SX_SCC_MASK. If it is shifted 10 bits to the right, the code coincides withcontrol codes of the format of messages for the BC which are represented as constants:

- DATA_BC_RT 		 - data transfer BC-RT;

- DATA_BC_RT_BRCST 	 - data transfer BC-RT in the broadcast mode;

- DATA_RT_BC 		 - data transfer RT-BC;

- DATA_RT_RT 		 - data transfer RT-RT;

- DATA_RT_RT_BRCST 	 - data transfer RT-RT in the broadcast mode;

- CTRL_C_A 		 - mode command of the format COMMAND-STATUS;

- CTRL_C_BRCST 	 - mode command of the format COMMAND in the broadcast mode;

- CTRL_CD_A 		 - mode command of the format COMMAND+DATA-STATUS;

- CTRL_CD_BRCST 	 - mode command of the format COMMAND+DATA in the broadcast mode;

- CTRL_C_AD 		 - mode command of the format COMMAND-STATUS+DATA.

If necessary it is possible to interrupt a chain of messages, having called mtstop function. This function has no parameters, and its call results in a zeroing of bit of automatic continuation for current start base. As a result the MT after the finishing of the current message stops and as usually, forms interrupt mtIntX. But, if at the moment of call mtstop in a bus channel there are no messages the MT remain in a state of the waiting of next message (which has not begin yet). If it is not required to wait for a message (for example, the program should complete operation) for immediate break of the Monitor it is possible to use mtreset function.

Except for the definition of signal interrupts there is one more way of tracing behind base from which the MT works at the moment. mtgetstate function returns a double word, which low 16 bits contain active base number, and high 16 bits - a current word of a state. Call of this function results in reset of the not serviced signal interrupt if it in this moment was, therefore it is necessary to combine with care mtgetstate function and possibility of the definition of signal interrupts.

The Remote Terminal

In RT mode DPRAM it is divided into subaddresses - sections in length of 32 words. Thus, to each subaddress in a RT, addressed by the Bus Controller, corresponds the appropriate memory in the RT. Subaddresses on receiving and subaddresses on transmission differ. In one RT it is accessible a maximum of 30 subaddresses (in the mode without the instrumentation bit, and in the mode with the instrumentation bit - 15 subaddresses). The storage areas for unused subaddresses (00000 and 11111 (bin.))) are used for storage of flag words (in RT mode with flags) and DATA for mode commands with DATA. The required memory size is 2 * (30 + 2) * 32 = 2048 words. If the memory size, installed on the device, exceeds 2К words (for example, 8К = 8192 words) all memory is divided on some pages size of 2К words which can be switched program. In this case devices either allow the separate definition of the page accessible on the side of the bus CHANNEL and the page accessible on the side of the PC (TMKX device), or provide the definition of the same working page for both the bus CHANNEL and the PC (TMK400, RTMK400 devices).

In RT mode the device generates two types of interrupt at least: interrupt on receiving of the mode command rtIntCmd and interrupt on error rtIntErr. Interrupt on receiving of the mode command is generated not for all commands, but only for what can not be executed by the device itself. Interrupt in error is generated in the event that the device has received a COMMAND, addressed to the given RT, and the error of the format of the message, an error of the Manchester code or a parity error is detected. Having read the device state word, it is possible to determine bits of the last COMMAND at which there was an error. The concrete reason of an error can not be determined.

By default, the RT work in the mode in which commands of data receiving/transmitting do not cause any interrupts. Old cards (TMK400, RTMK400, TMKMPC) had no possibility to form such interrupts in general. In new cards (TMKX/TMKXI) there is a possibility of enabling of the mode of generation of interrupts on data receiving/transmitting rtIntData. If interrupts rtIntData are not used, it is necessary to make program polling of the device depending on a mode of operation of the device for definition of the fact of data receiving/transmitting. If messages to a concrete RT are rare enough, it is possible to recommend either timer based RT state polling or attending of data transfers by the additional mode command which will call interrupt in the RT.

DOS

As the timer it is possible to use either the system timer with frequency 18.2 Hz or the timer which is included in IBM PC AT Real Time Clock subsystem which support functions are included in tmkml.c library.

All

Mode of operations of the RT either are defined by soldering or jumpers setting on the device (for devices TMK400, TMKMPC) or should be set program (for devices RTMK400, TMKX). The behaviour of the RT is determined by a combination of three modes which can be separately switched on or off:

- A mode of usage of the instrumentation bit (bit of RT_HBIT_MODE mode);

- A mode of operation with flags (bit of RT_FLAG_MODE mode);

- A mode of receiving of broacast messages (bit of RT_BRCST_MODE mode).

On the devices which are not accepting the program definition of mode of operations of the RT, the manufacturer installs modes of usage of the instrumentation bit and receiving of messages with the broadcast address. If necessary to change mode of operations of the RT, depending on concrete type of the device, it can be necessary to change jumper settimgs.

For the devices accepting the program definition of mode of operations of the RT, rtdefmode and rtgetmode functions are intended. rtdefmode function sets the mode, defined by a combination of bits RT_HBIT_MODE, RT_FLAG_MODE and RT_BRCST_MODE, for the selected device and writes it in an internal variable of the driver, and rtgetmode function reads this variable. At function call tmkconfig in this variable the default valuse (RT_HBIT_MODE|RT_BRCST_MODE) is always written, and function call rtreset results in setting of mode of operations of the RT according to value of this variable.

Besides some devices (TMKX) admit customization of the interrupts mode of the RT, in particular, the blocking of interrupts on bus jam, the blocking of interrupts on data receiving/transmitting, and also disconnecting of the interrupt request line of the device. For control of such devices constants are intended:

- RT_GENER1_BL - block interrupts on generation in BUS A;

- RT_GENER2_BL - block interrupts on generation in BUS B;

- RT_DATA_BL - block interrupts on data receiving/transmitting;

- TMK_IRQ_OFF - disconnect the interrupt request line of the device.

For customization of such devices in RT mode rtdefirqmode and rtgetirqmode functions are intended. rtdefirqmode function sets the mode defined by a combination of bits RT_GENER1_BL, RT_GENER2_BL, RT_DATA _BL, TMK_IRQ_OFF for the selected device and writes it in an internal variable of the driver, and rtgetirqmode function reads this variable. At function call rtreset in this variable the default value (RT_GENER1_BL|RT_GENER2_BL|RT_DATA_BL), specifying compatibility mode of all devices is always written.

The mode of usage of the instrumentation bit determines, whether the RT will react to command words only with the set instrumentation bit or on any command words (in this case the instrumentation bit expands a field of a subaddress and can receive any values). The mode of usage of the instrumentation bit should be set the same for all RTs in the bus CHANNEL! Instrumentation bit mode can be used for increasing of reliability of operation of the channel by means of introduction of distinctions between the command and status words having an identical sync signal (the instrumentation bit should be set in the command word and it is reset in status), and for simplification of operation of the Monitor of the channel. Turn off of the mode provides more flexibility of addressing by means of the extension of a field of a RT subaddress in the command word.

The mode of receiving of broacast messages determines, whether the RT will react to command words with the broadcast address (11111 bin). If the mode is switched on, the RT reacts as to messages with the RT address as to messages with broadcast address. If the mode is switched off the RT reacts only to the messages addressed directly to it. For different RTs in one channel it is possible independently to switch this mode on or off depending on system requirements.

The mode of operation with flags determines a way of arbitration of access to subaddresses in DPRAM between the channel and the PC. Thus one of two possible modes can be selected: the mode with flags or the mode without flags. More simple is the mode without flags; it can be applied in systems where loss of the data or a reuse of the data is allowed (for example, the BC reads out periodically a state of any sensor, or writes a state of any control, thus the BC with any periodicity accesses to the RT, and the processor of the PC of the RT in cycle reads or updates the data in DPRAM), or the schedule of transmissions on the bus CHANNEL is known beforehand precisely. In this mode on any command of the BC to receive or transmit data the RT receives the data and writes them in the indicated subaddress or reads the data from the indicated subaddress and transmits them in the bus CHANNEL. RT mode with flags should be used in systems where loss or a reuse of the data is inadmissible, or the schedule of transmissions on the bus CHANNEL is not known beforehand. In this mode for each subaddress (subaddresses on receiving and on transmission are different) there is the so-called flag word defining availability of the data in an appropriate subaddress. Flag words are located in DPRAM in the field of unused subaddresses.

Switching of the TMK device in RT mode with simultaneous reset of the device, and reinitialization of the mode, is carried out by rtreset function. Thus the zero working page in DPRAM is set.

For selection of working page in DPRAM rtdefpage function is intended. If the device allows to set page for the bus CHANNEL and page for the PC separately it is possible to use rtdefpagebus, rtdefpagepc functions, but call to these functions for the devices which are not having such possibility, will cause an erratic situation. rtgetmaxpage function returns maximum accessible number of page on the selected device (numbers of pages lay in a range from 0 up to this maximum number). To find out number of selected page it is possible to use rtgetpage function (numbers of separate pages for the bus CHANNEL and the PC are returned by rtgetpagebus and rtgetpagepc functions). It is necessary to switch pages in an operating time with care to be sure that there is no situation of switching of page during message from BUS CHANNEL, so  the moment of switching should be synchronized with operation of the BC. In most cases it is possible (and, probably, it is desirable) to work in a single page.

For operation with DPRAM the programmer at first should select a subaddress with rtdefsubaddr function. In limits of one page access to 30 subaddresses of receiving and 30 subaddresses of transmission is possible. The used range of subaddresses depends on a mode of operation of the bus CHANNEL - with the instrumentation bit (10000 - 11110 (bin)) or without the instrumentation bit (00001 - 11110 (bin)). The concrete mode is set by a jumper setting on the device or programmed by rtdefmode function and influences only operation of the RT (in the mode with the instrumentation bit the RT does not answer commands with the instrumentation bit equal 0). For selection of a subaddress of receiving or transmission it is possible to use constants RT_RECEIVE and RT_TRANSMIT. rtdefsubaddr function does not check a specified subaddress and a transmitting direction, and adjusts them by default, clearing "superfluous" bits, therefore the programmer should watch that "incorrect" subaddresses 00000 (bin) and 11111 (bin) were not set itself. To find out a selected subaddress it is possible to use rtgetsubaddr function, which a main usage - user's interrupt functions. This function returns one word, which low five bit contain a current subaddress, and the transmitting direction of a subaddress is shown by bit "T/R" which is equivalent to the same bit of the COMMAND (as it is located in a word and receives the same values). For selection of this bit it is possible to use constant RT_DIR_MASK, and the obtained value will be equal RT_RECEIVE for a subaddress of receiving and RT_TRANSMIT for a subaddress of transmission. After selection of a subaddress all operation with DPRAM occurs in this subaddress before selection of other subaddress with rtdefsubaddr.

DOS

At function call tmksave for the RT the current subaddress is saved by the driver so the programmer can work in other subaddresses, and by call tmkrestore the saved subaddress is restored as current.

All

In a selected subaddress rtputw, rtgetw functions can be used for writing, reading of words in DPRAM and rtputblk, rtgetblk functions can be used for writing, reading of blocks of words in DPRAM. With these functions in a selected subaddress of transmission the programmer prepares the information for transmission to the bus CHANNEL, and from a selected subaddress of receiving receives the information received from the bus CHANNEL.

In the mode without flags the program can access to any subaddress with which in this moment does not work a message of the bus CHANNEL at any moment. Two algorithms of operation are possible without flags, providing the resolution of conflicts at access in same subaddress of the bus CHANNEL and the PC. According to the first one program should determine availability of selected (by rtdefsubaddr function) subaddress with rtbusy function returning zero value if the selected subaddress is free, and non-zero if the selected subaddress is busy. The subaddress is busy if at the moment of rtbusy call a message in the bus CHANNEL adresses to this subaddress (subaddresses of receiving and transmission differ). If the subaddress is busy, it is necessary to wait for its release, again calling rtbusy and if it is free than in time no more than 16 us (at reading) or 20 us (at writing) after rtbusy call the program is obliged to read or write a word of the data and further to read or write them with cycle no more than 20 us. Other algorithm provides the locking of a subaddress rtlock function. This function has same parameters as for rtdefsubaddr function, and also leaves the subaddress specifed to it as selected. However, after call rtlock, the bus CHANNEL will not have access to a selected subaddress, and all calls to this RT subaddress will be answered with STATUS with set bit BUSY "Subsystem Busy", and the program can work with the locked subaddress in any rate during any time. Exception represents a case when rtlock is called in the moment when a message in the bus CHANNEL already works with a locked subaddress. In this situation message is not interrypted, and the locking of a subaddress is delayed up to the moment of finishing of the message. For detection of this situation it is necessary to use rtbusy function which will inform if the subaddress is busy until the end of the message. When the subaddress is succesfully locked, rtbusy always returns zero value. For unlocking of the locked subaddress rtunlock, rtlock, rtdefsubaddr functions can be used. After rtunlock function the current selected subaddress remains, but any subaddress is not locked. rtlock function unlocks a current subaddress, selects a new subaddress and locks it. rtdefsubaddr function unlocks a current subaddress and selects a new subaddress, not locking it. In every moment any single subaddress can be locked only.

Algorithm of operation with flags is the following. Transmitting side before data writing in DPRAM should check, that the flag of a selected subaddress is reset, only in this case it can to write the data and to set a flag of a subaddress. Receiving side before data reading from DPRAM should check, that the flag of a selected subaddress is set, only in this case it can to read the data and to reset a flag of a subaddress. Thus, at data transfer from the RT the device checks a flag word of an addressed subaddress of transmission, and if the flag is set (the new data in a subaddress are ready), reads the data from DPRAM and transmits them in the bus CHANNEL and if the flag is reset (the new data in a subaddress are not ready, reading from a subaddress on the side of the bus CHANNEL is forbidden), the RT answers with STATUS with set bit BUSY "Subsystem Busy ", the data from DPRAM is not read and is not transmitted in the bus CHANNEL. At receiving of the data in the RT the device checks a flag word of an addressed subaddress of receiving and if the flag is reset (the previous data in a subaddress are read by the PC), receives the data from the bus CHANNEL and writes them in DPRAM and if the flag is set (the previous data in a subaddress are not read yet in the PC, writing in a subaddress on the side of the bus CHANNEL is prohibited), receives the data from the bus CHANNEL, checking them on errors, but does not write them in DPRAM, and in the end the RT answers with STATUS with set bit BUSY "Subsystem Busy". At access to DPRAM on the side of the PC the program itself should read, check and write flags according to the described algorithm for all subaddresses with which it is going to work: to set after writing of the information in a subaddress of transmission for enabling of transmission and to wait for resetting, to reset for enabling of receiving in a subaddress of receiving and to wait for setting for reading the information from a subaddress. For this purpose rtsetflag, rtclrflag, rtgetflag functions are intended. rtgetflag function has same parameters as for rtdefsubaddr function, reads a flag word for a defined subaddress and selects this subaddress as a current selected subaddress. rtsetflag and rtclrflag functions set and reset a flag for a selected earlier subaddress (by rtdefsubaddr or rtgetflag functions). In addition to these functions rtgetflags and rtputflags functions are included in the driver, allowing to read and write at once some flag words and operating with the buffer array in RAM of the PC. After call of these functions the current selected subaddress is not defined, and its new definition, for example, with rtdefsubaddr function is necessary. The format of a flag word is given on fig. 4.

The programmer has possibility to control value of separate bit in STATUS for what rtsetanswbits and rtclranswbits functions are intended, allowing to set and reset the defined bits of STATUS. With rtgetanswbits function it is possible to find out a current state of these bit. For changing of bits it is possible to use constants:

- SREQ - bit "Service Request" is set if the subsystem of the RT requires any service on the BC; if it can be more than a single reason of setting of this bit, the concrete reason should be indicated in a vector word, which BC can receive on the command "Transmit Vector Word";

- BUSY - bit "Subsystem Busy" is set if in a subsystem any conditions exist during which a message from BUS CHANNEL can not be processed (for example, during DPRAM testing). At setting of this bit the device does not execute commands of the BC on data receiving/transmitting, answering only STATUS with set bit BUSY; if the bit was not set by program, the device can set it independently for separate commands of data message according to the algorithms of operation described above;

- SSFL - bit "Subsystem Flag" is set if in the subsystem of the RT any fault is detected that speaks the BC about probable invalidity of the data at message from the RT; the bit can be set and reset only program;

- RTFL - bit "Terminal Flag" is set if any fault in the TMK device or in the other equipment considered in the given system as a part of the terminal of the bus CHANNEL is detected, the device has no built-in hardware of selftesting, therefore the bit can be set and reset only program; the BC has possibility to mask this bit in STATUS with mode commands "Inhibit Terminal Flag Bit" and "Override Inhibit Terminal Flag Bit"; the concrete type of a fault can be indicated in a built-in test word, which BC can receive on the command "Transmit Built-In Test Word";

- DNBA - bit "Dynamic Bus Control Acceptance” is set in the RT if it has an ability at any moment to take up control of the channel (to become the next active BC) on the mode command "Dynamic Bus Control"; if the bit is not set by the program, the indicated command addressed to the RT produces STATUS with reset bit "Dynamic Bus Control Accepted”, that means the RT discarded the offer of the BC to receive control; if the bit is set by the program the RT answers on the indicated command with STATUS with the set bit "Dynamic Bus Control Accepted” and continues to answer such STATUSES all following commands of the BC (before to the command "Dynamic Bus Control" the bit in STATUS is not set even when bit DNBA set by the program); program reset of bit DNBA does not result in reset of bit "Dynamic Bus Control Accepted” in STATUS; to stop output of such STATUSES it is possible either to call rtreset function or from the BC send the mode command "Reset Remote Terminal".

For setting or clearing at once several bits by call of rtsetanswbits and rtclranswbits functions the listed constants can be ORed. Setting or clearing of selected bits does not influence the state of the others bits.

To write or read a DATA for mode commands with DATA ("Transmit Vector Word", "Transmit Built-In Test Word", " Synchronize with Data", "Selected Transmitter Shutdown", "Override Selected Transmitter Shutdown") rtputcmddata and rtgetcmddata functions of driver are intended. As parameters function receive the command for which it is necessary to write or read a DATA and also a DATA word for rtputcmddata function, rtgetcmddata function returns the read word as result. The command should contain the mode code itself (low five bit) and the “transmission/receiving” bit same as the “transmission/receiving” bit of the command word. It is possible to use constants of codes of commands:

CMD_DYNAMIC_BUS_CONTROL

CMD_SYNCHRONIZE

CMD_TRANSMIT_STATUS_WORD

CMD_INITIATE_SELF_TEST

CMD_TRANSMITTER_SHUTDOWN

CMD_OVERRIDE_TRANSMITTER_SHUTDOWN

CMD_INHIBIT_TERMINAL_FLAG_BIT

CMD_OVERRIDE_INHIBIT_TERMINAL_FLAG_BIT

CMD_RETURN_REMOTE_TERMINAL

CMD_TRANSMIT_VECTOR_WORD

CMD_SYNCHRONIZE_WITH_DATA_WORD

CMD_TRANSMIT_LAST_COMMAND_WORD

CMD_TRANSMIT_BUILT_IN_TEST_WORD

To determine a current state of the RT it is possible to use rtgetstate function which returns a word of a state (fig. 3). Low order 11 bit of a word of a state correspond to appropriate bits of the last COMMAND processed (or has been being processed at present) by the RT. Besides the word of a state contains bit of usage of a subaddress from the bus channel, bit "DPRAM access error" (only in old cards TMK400, RTMK400, TMKMPC), bit "Message Error in BUS CHANNEL". The bit of usage of a subaddress is set in the non flag mode if the subaddress selected byf the program at present, coincides with a subaddress, addressed by appropriate bits of the COMMAND. The “DPRAM access error" set bit can point appearance of a fault of the TMK device. The bit "Message Error in BUS CHANNEL" is set if the RT has detected in the message addressed to it an error of the format or an error of the Manchester code.

For the definition of the RT address in the bus CHANNEL rtdefaddress function is intended. To determine the earlier set address rtgetaddress function is intended, and if the device allows reading of the set address (for example, TMK400, RTMK400) the address is read from the device and if such possibility is absent the address is read from an internal variable of the driver.

As it was already told, the RT forms interrupts rtIntCmd, rtIntErr and rtIntData. For interrupt rtIntCmd the driver transmits the code of the command which includes 5 low bit of the command and “transmission/receiving” bit of the COMMAND as the parameter. For interrupts rtIntErr the driver transmits as the parameter a word of a state of the RT with the set bit of an error in the bus CHANNEL, and on other bits of a word of a state it is possible to determine the command, on which execution the error is detected. As in devices TMK400, RTMK400, TMKMPC, TMKX, TMKXI there is no buffering of interrupts, at continuous arrival with a minimum time of the commands, addressed to the same RT, the situation when the state information of interrupt is available to reading from the device less, than 20 us, is possible. If the system has no enough time to call the interrupt handler of the driver for this time, appearance of ambiguity is possible at interrupt processing. Usually it can be missing interrupt, besides on cards TMK400, RTMK400, TMKMPC it is possible to get an interrupt rtIntErr without the set bit of an error (bits of a word of a state in this case contain bits of a new COMMAND).

DOS

Before call of user's functions the driver saves the selected device number and a current subaddress of the device which have caused interrupt. If necessary to restore at the end of interrupt a state of the device and the driver, rtrestore function should be called.

Multi-address RT

Linux, Windows

Since v2.00 version the driver supports operation with cards TX6/TE6 in multi-address RT mode (MRT). In this mode on one physical device of a multi-address RT 4 virtual RTs (VRT) with different addresses in the channel are realized. If for any device at the configuration of the driver set the MRT mode, the list of devices of the driver will contain as this device with number defined in the registry (configuration) as additional 4 VRTs with numbers, assigned by the driver.

For operation with MRT new functions are entered:

TMK_DATA rtenable(TMK_DATA rtEnable);

int mrtgetmaxn();

DWORD mrtconfig(int mrtNumber);

int mrtselected();

TMK_DATA mrtgetstate();

void mrtdefbrcsubaddr0();

int mrtreset();

- rtenable function is included which main purpose is to enable or disable operation of a current VRT in the MRT. However with some limitations this function can be used and for usual RTs. Exception is the RT on the TMK400 and TMKMPC which can not be disabled by this function. The RT is enabled on call rtenable(RT_ENABLE), and is disabled by call rtenable(RT_DISABLE). It is possible to receive a current state (RT_ENABLE or RT_DISABLE) through call rtenable(RT_GET_ENABLE).

Besides the algorithm of operation of rtreset and rtdefaddress functions for all RTs is modified. If rtreset is called first time after tmkconfig or bcreset or mtreset the device is switched to RT mode, but remains disabled as after call rtenable(RT_DISABLE). The device switches on only after the definition of its address through rtdefaddress. Following rtreset calls do not change a enabled/disabled state.

mrtgetmaxn function returns maximum possible number of the MRT. This number is equal to maximum possible number of the physical device in the driver. That is for the driver designed for operation with 8 devices (with 0 on 7) mrtgetmaxn will return 7. tmkgetmaxn function without the MRTs configured in the registry (configuration) works as well. At presence of the MRT the driver will assign to VRTs logical numbers, since 8, and tmkgetmaxn will return maximum assigned number of the VRT. That is, if one MRT with 4 VRTs is configured tmkgetmaxn returns 11.

mrtconfig function should be called instead of tmkconfig for the MRT if the program is written specially for operation with MRT. This function configures a MRT and all VRTS which are included in it, for operation with the process which has called mrtconfig. mrtconfig function returns a double word, which low 16 bits is a first number of the VRT, containing in this MRT, and high 16 bit - quantity of the VRTs, containing in this MRT. At an error (for example if defined number is not a MRT number) the function will return zero value.

For operation with multi-address RTs two ways are possible. It is possible to capture a MRT and all VRTS included in it by call mrtconfig. Other way - to not call mrtconfig for the MRT and to call tmkconfig for each concrete VRT. It is necessary to remember, that the MRT has some common resources for all embedded VRTs, for example, the arbiter of access to a subaddress of the DPRAM (used in rtlock, rtunlock, rtbusy functions), bits of mode of operations (rtdefmode, rtgetmode, rtdefirqmode, rtgetirqmode functions), the register of the hardware reset (rtreset, mrtreset functions). If different VRTs are captured by different processes through tmkconfig calls, these functions called for one VRT will influence operation of the others. rtreset function if it is called for the VRT, will perform complete hardware reset of the MRT only in the case if this VRT is captured only, and the MRT and other VRTs are not captured by any process. mrtreset function will perform the hardware reset of the MRT and all included in it VRTs in any case.

Captured through mrtconfig or tmkconfig the MRT and/or the VRT are selected for operation, as well as earlier, by tmkselect function.

mrtselected function allows to find out number of a selected MRT for a current selected VRT. For usual RTs of tmkselected and mrtselected function will return the same value. For the VRT tmkselected will return number of the VRT, and mrtselected - number of the MRT.

mrtreset function performs the hardware reset and initialization of the MRT and all embedded VRTs.

For the VRT and the MRT bcreset and mtreset functions can not be called, therefore in universal programs if they can be used on computers with installed MRT, it is desirable to check a return code from these functions. For usual devices the bcreset return code will be zero. For the MRT/VRT bcreset will return a non-zero error code.

mrtgetstate function allows to read the word of a state of the MRT (see the description on the MRT mode). rtgetstate function used for reading of the word of a state of usual RTs, for the VRT will read the virtual word of a state formed by the driver.

mrtdefbrcsubaddr0 function allows to select as a current subaddress of the MRT or the lowest number VRT a subaddress in which come all broadcast messages (see the description on the MRT mode). It is necessary to remember, that even if the lowest number VRT is disabled, but there is any another enabled VRT in structure of the same MRT, the data from messages with the broadcast address will arrive in this subaddress.

APPENDIX

Figures and Tables

Formats of messages with the indication of allocation

of the messages in DPRAM base of the BC

for message length of 32 data words

| Offset in base: | 00 | 01 | 02 |  | … |  | 32 | 33 | 34 | 35 |  | … |  | 63 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BC-RT | CMD | DAT | DAT | … | … | DAT | DAT | STA |  |  |  |  |  |  |
| RT-BC | CMD | STA | DAT | DAT | … | … | DAT | DAT |  |  |  |  |  |  |
| RT-RT | CM1 | CM2 | ST2 | DAT | DAT | … | … | DAT | DAT | ST1 |  |  |  |  |
| COMMAND-STATUS | CMD | STA |  |  |  |  |  |  |  |  |  |  |  |  |
| COMMAND-STATUS+ DATA | CMD | STA | DAT |  |  |  |  |  |  |  |  |  |  |  |
| COMMAND+ DATA- STATUS | CMD | DAT | STA |  |  |  |  |  |  |  |  |  |  |  |
| BC-RTbrc | CMD | DAT | DAT | … | … | DAT | DAT |  |  |  |  |  |  |  |
| RT-RTbrc | CM1 | CM2 | ST2 | DAT | DAT | … | … | DAT | DAT |  |  |  |  |  |
| COMMANDbrc | CMD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COMMAND+ DATAbrc | CMD | DAT |  |  |  |  |  |  |  |  |  |  |  |  |

The BC-RT, the RT-BC, the RT-RT, the BC-RTbrc, the RT-RTbrc are formats of data transfer commands, and the COMMAND-STATUS, the COMMAND-STATUS+DATA, the COMMAND+DATA-STATUS, the COMMANDbrc, the COMMAND+DATAbrc are formats of  mode commands.

NOTE. DATAs in modes of the RT-BC, the RT-RT, the COMMAND-STATUS+DATA and STATUSes in all modes are written in DPRAM after start of message.

Fig. 1.

FORMATS of WORDS

Command Word (COMMAND)

15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 00

---+--+--+--+--+--+--+--+--+--+--+--+--+--+--+---

+--------------+--+--+-----------+--------------+

¦     ADR      ¦K ¦B |  SA/CI    ¦    N/COP     ¦

+--------------+--+--+-----------+--------------+

ADR - an RT address field:

00000-11110 - the RT address;

11111 (31) - the broadcast address;

К - the “transmission/receiving” bit:

0 - the RT receives the information (RT_RECEIVE),

1 - the RT transmits the information (RT_TRANSMIT);

B - the instrumentation bit:

0/1 - most significant bit of a subaddress in the mode without instrumentation bit,

1 - in the mode with the instrumentation bit (HBIT_MASK);

SA/CI - subaddress/mode field (together with bit B):

00000, 11111 - sets the mode command in field N/COP in the mode w/o instrumentation bit,

11111 - sets the mode command in field N/COP in the mode with the instrumentation bit (CI_MASK),

00001-11110 - sets a subaddress in the mode without instrumentation bit,

10000-11110 - sets a subaddress in the mode with the instrumentation bit;

N/COP - number of data words the/mode code:

00000 - 32 words in a data transfer mode,

00001-11111 - 1-31 word in a data transfer mode,

00000-11111 - the mode code in mode command.

Data Word (DATA)

15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 00

---+--+--+--+--+--+--+--+--+--+--+--+--+--+--+---

-------------------------------------------------

¦                    Data                       ¦

-------------------------------------------------

Status Word (STATUS)

15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 00

---+--+--+--+--+--+--+--+--+--+--+--+--+--+--+---

+--------------+--+--+--+--------+--+--+--+--+--+

¦    ADR       ¦A ¦B ¦C ¦X  X  X ¦D ¦E ¦F ¦G ¦H ¦

+--------------+--+--+--+--------+--+--+--+--+--+

ADR - address of answering RT:

00000-11110 - the RT address (ADDR_MASK);

A - “Message Error” bit (ERROR_MASK);

В - “Instrumentation” bit (HBIT_MASK);

C - "Service Request" bit (SREQ_MASK);

XXX - reserved bits (000) (NULL_MASK);

D - "Broadcast Command Received" bit (BRCST_MASK);

E - "Subsystem Busy" bit (BUSY_MASK);

F - "Subsystem Flag" bit (SSFL_MASK);

G - "Dynamic Bus Control Acceptance" bit (DNBA_MASK);

H - "Terminal Flag" bit (RTFL_MASK);

Fig 2.

Word of a State of the RT Format.

15  14  13  12  11  10  09  08  07  06  05  04  03  02  01  00

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+

¦ X ¦EM ¦ X ¦ER ¦BP ¦T/R¦      SA/CI        ¦       N/COP       ¦

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+

+-----------------------v-------------------+

Last Command Word Bits

NOTE: the word of a state of the RT is returned by rtgetstate function, and also transmitted as the parameter of interrupts rtIntErr, rtIntData.

T/R - “transmission/receiving” bit of the last COMMAND (RT_DIR_MASK);

SA/CI - subaddress/mode field of the last COMMAND (SUBADDR_MASK, CI_MASK);

N/COP - word count/mode code field the last COMMAND (NWORS_MASK, CMD_MASK);

BP - the bit of usage of a selected subaddress in the mode without flags, it is set if the selected subaddress is accessed from BUS CHANNEL at the moment;

ER - bit "DPRAM Access Error", the setting of this bit can point a fault in the device (this bit can be set only in cards TMK400, RTMK400, TMKMPC);

EM - bit "Message Error" is set if in the message addressed to the RT there is an error of the format of the message, an error of the Manchester code or a parity error (RT_ERROR_MASK).

Fig. 3.

The Format of a Flag Word of the RT.

15  14  13  12  11  10  09  08  07  06  05  04  03  02  01  00

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+

¦FLG¦ X ¦ X ¦ X ¦ X ¦T/R¦        SA         ¦         N         ¦

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+

+-----------------------v-------------------+

Command Word Bits

NOTE: at writing of a flag word in DPRAM with rtputflags function it is necessary to set only bit of flag FLG, and at the analysis of flag words read by rtgetflag, rtgetflags functions it is possible to analyse fields T/R, SA, N which are filled by the device at writing of a flag word according to the message from BUS CHANNEL. For example, it is possible to find out the concrete number of the words which have been received/transmitted in/from the subaddress. rtsetflag function writes in a flag word constant 8000 (hex), and rtclrflag function writes constant 0000.

T/R - “transmission/receiving” bit of the COMMAND (RT_DIR_MASK);

SA/CI - bits of a subaddress field of the COMMAND (SUBADDR_MASK);

N/COP - bits of a number of words field of the COMMAND (NWORS_MASK);

FLG - bit of a flag of availability of the data (RT_FLAG, RT_FLAG_MASK).

Fig. 4.

The Format of a Word of a State of the Message (MT).

15  14  13  12  11  10  09  08  07  06  05  04  03  02  01  00

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+

¦BUS¦ME ¦      SCC      ¦K1 ¦K2 ¦ X ¦ X ¦G1 ¦G2 ¦IB ¦    ERR    ¦

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+

NOTE: the word of a state of the message of the MT is returned by mtgetsw function for current selected base, and also transmitted as the parameter of the mtIntX interrupt for the last received message (the word of a state transmitted as a mtIntX parameter cannot be received via mtgetsw function later).

BUS - bit of the BUS number on which the message is received (SX_BUS_MASK);

ME - bit of the integrated tag of an error in the message (SX_ME_MASK);

SCC - a field of the code of the format of the message, at its shift on 10 bits to the right coincides with the control code of the BC (SX_SCC_MASK);

K1 - bit of tag of an error in the first COMMAND (SX_K1_MASK);

K2 - bit of tag of an error in the second COMMAND (SX_K2_MASK);

G1 - bit of tag of bus jam in BUS A (SX_G1_MASK);

G2 - bit of tag of bus jam in BUS B (SX_G2_MASK);

IB - bit of tag of set bits in STATUS (SX_IB_MASK);

ERR - a field of an error code of the message (SX_ERR_MASK).

Fig. 5.

The Coding of Field SA/CI in Commands of the Bus CHANNEL. Table 1.

+-------------+-------------------------------------------------------+

¦ SA/CI field ¦       Subaddress or Mode Command                      ¦

+-------------+-------------------------------------------------------+

¦ 00000,11111 ¦Mode Command Indicator.                                ¦

¦    00001    ¦Subaddress.                                            ¦

¦     ...     ¦                               ¦

¦    11110    ¦                                                       ¦

+-------------+-------------------------------------------------------+

The Coding of Field N/COP in Mode Commands of the Bus CHANNEL. Table 2.

+-----------+----------------------------+---------+--------+---------+

¦ N/COP Code¦         Command            ¦   Bit   ¦  Data  ¦Broad-   ¦

¦           ¦                            ¦   T/R   ¦  word  ¦cast     ¦

¦           ¦                            ¦         ¦        ¦address  ¦

+-----------+----------------------------+---------+--------+---------+

¦            Mode Commands of COMMAND-STATUS Format                   ¦

+-----------+----------------------------+---------+--------+---------+

¦ 00000     ¦Dynamic Bus Control         ¦    1    ¦    -   ¦    -    ¦

¦ 00001     ¦Synchronize                 ¦    1    ¦    -   ¦    +    ¦

¦ 00010     ¦Transmit Status Word        ¦    1    ¦    -   ¦    -    ¦

¦ 00011     ¦Initiate Self-Test          ¦    1    ¦    -   ¦    +    ¦

¦ 00100     ¦Transmitter Shutdown        ¦    1    ¦    -   ¦    +    ¦

¦ 00101     ¦Override transmitter Shutdow¦    1    ¦    -   ¦    +    ¦

¦ 00110     ¦Inhibit Terminal Flag Bit   ¦    1    ¦    -   ¦    +    ¦

¦ 00111     ¦Override Inhibit Terminal   ¦    1    ¦    -   ¦    +    ¦

¦           ¦Flag Bit                    ¦         ¦        ¦         ¦

¦ 01000     ¦Reset Remote Terminal       ¦    1    ¦    -   ¦    +    ¦

¦ 01001     ¦Reserved                    ¦    1    ¦    -   ¦    -    ¦

¦ . . .     ¦                            ¦         ¦        ¦         ¦

¦ 01111     ¦Reserved                    ¦    1    ¦    -   ¦    -    ¦

+-----------+----------------------------+---------+--------+---------+

¦            Mode Commands of COMMAND+DATA-STATUS and                 ¦

¦            COMMAND-STATUS+DATA Formats                              ¦

+-----------+----------------------------+---------+--------+---------+

¦ 10000     ¦Transmit Vector Word        ¦    1    ¦    +   ¦    -    ¦

¦ 10001     ¦Syncronize with Data        ¦    0    ¦    +   ¦    +    ¦

¦ 10010     ¦Transmit Last Command Word  ¦    1    ¦    +   ¦    -    ¦

¦ 10011     ¦Transmit Built-In Test Word ¦    1    ¦    +   ¦    -    ¦

¦ 10100     ¦Selected Transmitter        ¦    0    ¦    +   ¦    +    ¦

¦           ¦Shutdown                    ¦         ¦        ¦         ¦

¦ 10101     ¦Override Selected Trasmitter¦    0    ¦    +   ¦    +    ¦

¦           ¦Shutdown                    ¦         ¦        ¦         ¦

¦ 10110     ¦Reserved                    ¦    X    ¦    +   ¦    -    ¦

¦ . . .     ¦                            ¦         ¦        ¦         ¦

¦ 11111     ¦Reserved                    ¦    X    ¦    +   ¦    -    ¦

+-----------+----------------------------+---------+--------+---------

NOTE. X - arbitrary value of bit.

At creation of commands on the BC and their processing in the RT it is necessary to take into account a state of “transmission/receiving” bit. The driver always handles commands as words consisting of the instruction code (low 5 bit) and “transmission/receiving” bit. Here is the description of commands and their implementation on the TMK devices.

Commands "Synchronize" and " Synchronize with Data" are intended for realization of synchronization of operation of the BC and the RT.

The command "Transmit Vector Word" is coupled to a “Service Request” bit in STATUS and is used for definition of kind of service if for the terminal there are more than one kind of a service request. The program of the RT should set a vector word through rtputcmddata function in advance.

The command "Dynamic Bus Control” offers the RT to become new BC, having received control of the channel. If the RT is able to do that, it should answer STATUS with the set bit "Dynamic Bus Control Acceptance". The program of the RT, capable to switch the device in BC mode and further to control the channel, should in advance set with rtsetanswbits(DNBA) function bit "Ready to get in control of the channel". After receiving the indicated command such RT sets in each STATUS bit "Dynamic Bus Contro Acceptance". If for any reason it is necessary to continue operation in RT mode for termination of output of the set bit in STATUS the program of the RT should initialize the RT with rtreset function. For cancellation of operation of the command by a BC it is possible to use the mode command "Reset Remote Terminal” to reset the RT.

The BC can use the command "Reset Remote Terminal" for initialization of a selected RT. The command is executed by the device hardware and does not cause interrupt in the PC. In multi-address RTs and in future possible implementations of the RT the command can cause interrupt.

The command "Transmit Last Command Word" allows to determine the last valid COMMAND, received and executed by the RT prior to the given command. It does not change bits of a status word of the RT that allows to use it in operations on error detection and restoring, not influencing on STATUS that could bring in new errors. The given command is executed by the device hardware and does not cause interrupt in the PC.

The command "Transmit Status Word" also does not change bits of STATUS and allows the Bus Controller to obtain the refreshed data on the information passing on the bus CHANNEL. The given command is executed by the device hardware and does not cause interrupt in the PC.

On the command "Initiate Self Test" the program should check the equipment of the terminal which is the TMK device itself and other equipment which in the given system can be considered as a part of the terminal of the bus CHANNEL. This command can be used both at initialization of the system, and in procedures of restoring after refusal of the bus CHANNEL. The device does not contain any built-in resources of the self test, therefore the indicated command should be performed by program. After execution of self test the program of the RT in the case of fault should set "Terminal Flag" bit of STATUS with rtsetanswbits(RTFL) call, and call rtputcmddata to set a word of built-in test.

The command "Inhibit Terminal Flag Bit" can be used for setting of appropriate bit of STATUS in a state "normal", irrespective of the real state of an addressed RT. It allows to prevent repeating interrupts in the system on error handling and restoring when refusal is detected and the system has adapted. This command disables the subsequent error messages which should arise at usage of Terminal Flag Bit in each subsequent STATUS. The given command is executed by the device hardware and does not cause interrupt in the PC.

On the command "Override Terminal Flag Bit" effect of the previously described command is removed, allowing Terminal Flag bit in STATUS to display the real state of the terminal. The given command is executed by the device hardware and does not cause interrupt in the PC.

Commands "Transmitter Shutdown" and "Override Transmitter Shutdown" are used in systems with dual redundant bus. Having received such command on one BUS, the RT disables or enables the transmitter of other BUS. The status word is transmitted in the BC through a bus on which the command has arrived. The given commands are executed by the device hardware and do not cause interrupt in the PC.

On the command "Transmit Built-In Test Word" the RT answers STATUS with the attached data word which is the code of result of built-in test. This command allows the BC to receive the information on a possible defect cause of the RT. The command should not be used for obtaining the information on a fault of a subsystem. The BC usually uses the given command if in STATUS there is a set bit "Terminal Flag". The word of built-in test should be preliminary defined by the program of the RT with rtputcmddata function.

(c) “ELCUS Electronic Company”JSC, 1995,2005.